CUT2FILL

Connect. Reuse. Sustain.

Complete Business Case

Business Plan | Regulatory Framework | Feasibility Study
CSIRO Kick-Start Grant Application | MVP Specification

Prepared by: Archers Group Pty Ltd

Date: 23 March 2026

Version: 1.0

Classification: Commercial in Confidence

# Table of Contents

1.  Business Plan

2.  QLD Regulatory Compliance Framework

3.  Feasibility Study & CSIRO Kick-Start Brief

4.  SEQ Data Sources & Integration Strategy

5.  MVP Technical Specification

6.  Cost Intelligence & Material Specification Engine

7.  Material Tracking, Compliance Automation & Vision

# Cut2Fill — Business Plan

Version: 1.0

Date: 23 March 2026

Prepared by: Archers Group Pty Ltd

ABN: [Insert]

Domain: cut2fill.com.au (acquired)

## 1. Executive Summary

Cut2Fill is a digital platform that connects construction projects generating surplus earthworks material with projects that need it — eliminating unnecessary landfill disposal, reducing truck movements, and streamlining regulatory compliance for material movement across South East Queensland (SEQ).

The platform addresses a $380M+ annual problem: SEQ's construction industry generates approximately 3.4 million tonnes of construction and demolition (C&D) waste annually (23.2% of QLD's total waste stream), with an estimated 571,000 tonnes of reusable material still going to landfill. Since 1 July 2023, the removal of the clean earth levy exemption means every tonne of reusable fill sent to landfill now costs the industry $115/tonne (rising to $145/tonne by 2027-28) — a cost that didn't exist two years ago.

Cut2Fill is not a classifieds board for free dirt. It is a compliance-first material exchange platform designed for professional earthworks at scale (minimum 100m³), targeting government, tier 1-3 contractors, and licensed facilities. The platform integrates regulatory guidance, fire ant biosecurity zone awareness, registered facility mapping, and material classification compliance into every transaction.

### The Opportunity

$65.7M/year in avoidable waste levy costs on reusable material (571,000t × $115/t)

No existing End of Waste (EOW) code for soil/fill in Queensland — a regulatory gap that forces conservative disposal decisions

Brisbane 2032 Olympics driving unprecedented SEQ construction activity

Fire ant biosecurity compliance adds complexity and cost to every soil movement — no digital tool currently streamlines this

Government actively seeking circular economy solutions (QLD Waste Management and Resource Recovery Strategy, Resource Recovery Industry Development Program funding up to $10M)

### Revenue Model

| Phase | Revenue Stream | Target |
| --- | --- | --- |
| Year 1 | Free platform — user acquisition | 200 registered companies |
| Year 2 | Premium listings, verified supplier badges, API access | $180K ARR |
| Year 3 | Transaction facilitation, compliance documentation, government data licensing | $750K ARR |

## 2. Problem Statement

### 2.1 The Waste Problem

South East Queensland's construction boom (driven by population growth of 90,000+/year, Brisbane 2032 infrastructure, and Cross River Rail) generates enormous volumes of excavated material. Currently:

3.4 million tonnes of C&D waste generated annually in QLD

571,000 tonnes of reusable material still going to landfill

Waste levy of $115/tonne (2025-26) applies to ALL material entering licensed landfills, including clean earth since the exemption removal on 1 July 2023

No centralised platform connects material generators with material receivers

Project managers rely on phone calls, word of mouth, and personal networks

### 2.2 The Compliance Problem

Moving earthworks material in Queensland involves overlapping regulatory requirements that are difficult for non-specialists to navigate:

| Regulation | Trigger | Consequence of Non-Compliance |
| --- | --- | --- |
| EP Act 1994 s.319 | General environmental duty — ALL earthworks | Civil and criminal penalties |
| EP Act 1994 s.424-425 | Moving soil from EMR/CLR listed land | 100 penalty units ($16,692) per offence |
| Biosecurity Act 2014 s.23 | Moving soil within/between fire ant zones | 750 penalty units ($125,175) individuals; 3,750 ($625,875) corporations |
| EP Regulation 2019 ERA 60 | Disposing of waste at unlicensed site | Prosecution, remediation orders |
| State Planning Policy (ASS) | Excavation ≥100m³ below 5m AHD | Stop-work orders, remediation |
| Local government | Fill placement without approval | Compliance notices, fines |

The penalty for moving soil across a fire ant biosecurity zone boundary without a Biosecurity Instrument Permit (BIP) is $125,175 for an individual and $625,875 for a corporation. Yet there is no digital tool that alerts a project manager when a material exchange would cross zone boundaries.

### 2.3 The Information Problem

Critical data exists but is fragmented across multiple government systems:

Environmental Authorities Register — lists all ERA-licensed facilities but has no coordinates (text addresses only)

QLD Waste Facilities Dataset — has coordinates but last updated October 2018

Mining permits — available via ArcGIS REST API but requires GIS expertise to query

Fire ant zones — updated monthly via QLD Open Data but not integrated into any industry tool

EMR/CLR registers — searchable only via DES website, no API

No single platform consolidates this information into an actionable tool for construction professionals.

## 3. Solution

### 3.1 Platform Overview

Cut2Fill is a web-based platform (responsive, mobile-ready) with three core capabilities:

1. Material Exchange

Post listings for material available (surplus) or material wanted (demand)

Minimum 100m³ per listing — professional scale only

Smart matching algorithm considers material type, volume, location proximity, date overlap, and zone compatibility

Two pricing tiers: Free (material available at no cost) or Quote Required (commercial arrangement)

2. Compliance Intelligence

Automatic fire ant biosecurity zone checking on every listing

Zone crossing warnings when matching listings across different biosecurity zones

Material classification guidance with QLD-specific regulatory references

Links to relevant permits, forms, and government resources

Educational content for each material type (EP Act requirements, ASS management, contaminated land procedures)

3. Registered Facility Mapping

All SEQ quarries (from QLD Mining Permits ArcGIS REST API)

Licensed landfills and transfer stations (from QLD Waste Facilities dataset + EA Register)

Licensed soil treatment facilities (ERA 53, 54, 55, 60 filtered from EA Register)

PFAS treatment facilities

C&D recyclers

Key Resource Areas (KRA) overlay

Fire ant biosecurity zone overlay (from QLD Open Data, updated monthly)

### 3.2 Target Users

| User Type | Need | Cut2Fill Value |
| --- | --- | --- |
| Tier 1-3 Contractors | Dispose of surplus material quickly, cheaply, compliantly | Find receivers near their site, avoid $115/t levy, compliance guidance |
| Civil Earthworks Companies | Source fill material for projects | Find verified supply, reduce virgin quarry dependence |
| Local Councils | Monitor material flows, support circular economy targets | Dashboard data, facilitate reuse within LGA |
| State Government (DES, TMR, BQCC) | Track material movements, enforce compliance | Platform data informs policy, zone compliance built-in |
| Licensed Facilities | Attract customers, fill capacity | Permanent listings, verified badges, visibility |
| Developers | Manage earthworks costs on large projects | Find competitive fill/disposal options with compliance certainty |

### 3.3 What Cut2Fill Is NOT

Not a marketplace for small residential loads (<100m³)

Not a waste broker or transport company

Not a testing laboratory or certification body

Not a permit issuer — we provide guidance and links to government services

## 4. Market Analysis

### 4.1 Market Size

SEQ Construction Earthworks Market:

| Metric | Value | Source |
| --- | --- | --- |
| SEQ population growth | ~90,000/year | QLD Government Statistician |
| C&D waste generated (QLD) | 3.4M tonnes/year | QLD Waste Data Report |
| C&D waste to landfill | 571,000 tonnes/year | QLD Waste Strategy |
| Waste levy cost on reusable material | $65.7M/year | 571,000t × $115/t |
| Active construction sites SEQ (est.) | 2,000-3,000 | ABS Building Approvals |
| Major infrastructure projects (2025-2030) | 50+ ($50B+ pipeline) | Infrastructure Australia |

Key infrastructure projects driving demand:

Brisbane 2032 Olympic & Paralympic Games venues and athlete village

Cross River Rail (operational 2025, legacy works continuing)

M1 Pacific Motorway upgrades

Inland Rail (QLD section)

Brisbane Metro

Gold Coast Light Rail Stage 4

Sunshine Coast Airport expansion

Multiple major residential subdivisions (Springfield, Ripley, Yarrabilba, Caboolture West)

### 4.2 Competitive Landscape

| Competitor | Type | Weakness |
| --- | --- | --- |
| ReadyFill (readyfill.com.au) | WordPress classifieds for free fill | Small loads, no compliance, no mapping, no matching, residential focus |
| Dial Before You Dump | QLD phone service | Phone-only, no digital platform, no matching |
| iSeekPlant / PlantMiner | Equipment hire marketplace | Equipment focus, not material exchange |
| UrbanMine (Canada) | Material exchange platform | Not Australian, no QLD compliance |
| Word of mouth / phone calls | Industry default | No traceability, no compliance, no data |

Cut2Fill's moat: QLD-specific regulatory intelligence, fire ant zone integration, registered facility data, and government partnership positioning. No competitor has any of these.

### 4.3 Regulatory Tailwinds

Waste levy escalation — $115/t (2025-26) → $125/t (2026-27) → $145/t (2027-28). Every year, the economic case for reuse strengthens.

Clean earth exemption removal (1 July 2023) — previously, clean earth was exempt from the waste levy. Its removal created an entirely new cost pressure that didn't exist before.

QLD Waste Management and Resource Recovery Strategy — sets targets for C&D waste diversion.

Resource Recovery Industry Development Program — government funding up to $10M for C&D waste solutions.

No EOW code for soil/fill — this regulatory gap means there is no streamlined pathway for declaring excavated soil as "not waste." Cut2Fill can advocate for and facilitate the development of such a code.

Brisbane 2032 sustainability commitments — the Olympics bid included commitments to circular economy and sustainable construction.

## 5. Technology & Data Strategy

### 5.1 Architecture (MVP)

Frontend: Single-page application (HTML/CSS/JavaScript)

Leaflet.js for mapping (open source, no API key required)

Responsive design for field use on tablets/phones

No framework dependency for MVP — vanilla JS for speed and simplicity

Backend (Phase 2):

Node.js or Python (FastAPI) REST API

PostgreSQL with PostGIS for spatial queries

Authentication (email/password + optional Google OAuth)

Listing CRUD, matching algorithm, notification system

Data Integration:

| Source | Method | Refresh |
| --- | --- | --- |
| Fire ant zones | QLD Open Data GPKG/SHP download or ArcGIS REST API (Layer 65) | Monthly |
| Quarries/mining permits | ArcGIS REST MapServer (MinesPermitsCurrent) | Live query |
| Key Resource Areas | QLD Open Data SHP + ArcGIS REST | Quarterly |
| Waste facilities | QLD Open Data CSV (CKAN API) | On update |
| Environmental Authorities | QLD Open Data XLSX + geocoding | Monthly |
| BCC transfer stations | BCC Open Data GeoJSON API | Quarterly |

### 5.2 Matching Algorithm

The matching engine considers:

Material compatibility — type match (e.g., clean fill available → clean fill wanted)

Volume overlap — minimum viable exchange quantity

Proximity — distance between sites (driving distance, not straight line — Phase 2)

Date overlap — availability windows must intersect

Zone compatibility — flag if exchange crosses fire ant biosecurity zone boundary

Compliance requirements — flag if material type requires permits (contaminated soil, ASS)

### 5.3 Data Moat

Over time, Cut2Fill accumulates:

Historical material flow data across SEQ

Pricing intelligence (market rates for different materials)

Seasonal patterns in supply and demand

Transport corridor utilisation

Facility capacity and utilisation rates

This data becomes increasingly valuable to government for policy making and to industry for project planning.

## 6. Go-to-Market Strategy

### Phase 1: MVP & Validation (Months 1-6)

Goal: 50 registered companies, 200 active listings

Launch free platform with sample data demonstrating capability

Direct outreach to 20 civil earthworks companies in SEQ (Archers Group network)

Present to CCIQ (Chamber of Commerce & Industry QLD) construction committee

Present to CCF QLD (Civil Contractors Federation) members

Seek endorsement from 2-3 councils (start with BCC and Logan)

Key milestone: First real material exchange facilitated through the platform

### Phase 2: Government Partnership (Months 6-12)

Goal: Government endorsement, 200 companies, platform with real data

Formal approach to QLD Department of Environment and Science (DES)

Partnership discussion with Biosecurity Queensland (BQCC) — integrate official zone data

Present to SEQ Council of Mayors

Submit data to support development of an EOW code for clean fill

Apply for Resource Recovery Industry Development Program funding

### Phase 3: Revenue & Scale (Months 12-24)

Goal: Revenue generation, state-wide expansion

Launch premium features (verified supplier badges, priority listing, API access)

Transaction facilitation (optional — Cut2Fill manages compliance documentation)

Expand beyond SEQ to QLD-wide, then nationally

Government data licensing agreements

## 7. Financial Projections

### 7.1 Costs

| Item | Year 1 | Year 2 | Year 3 |
| --- | --- | --- | --- |
| Development (platform) | $40,000 | $60,000 | $80,000 |
| Hosting & infrastructure | $3,600 | $7,200 | $14,400 |
| Data integration & maintenance | $5,000 | $12,000 | $20,000 |
| Marketing & outreach | $10,000 | $25,000 | $40,000 |
| Personnel (part-time → full-time) | $0* | $80,000 | $160,000 |
| Legal & compliance | $5,000 | $8,000 | $10,000 |
| Total | $63,600 | $192,200 | $324,400 |

*Year 1 development via Archers Group internal capacity + AI-assisted development

### 7.2 Revenue

| Stream | Year 1 | Year 2 | Year 3 |
| --- | --- | --- | --- |
| Platform fees (free) | $0 | — | — |
| Premium listings ($50/month) | — | $60,000 | $120,000 |
| Verified supplier badges ($200/year) | — | $20,000 | $60,000 |
| API access (councils, large contractors) | — | $50,000 | $150,000 |
| Transaction facilitation (1% of matched value) | — | $50,000 | $250,000 |
| Government data licensing | — | — | $170,000 |
| Total | $0 | $180,000 | $750,000 |

### 7.3 Funding Strategy

| Source | Amount | Timing | Status |
| --- | --- | --- | --- |
| Archers Group internal | $40,000 | Immediate | Committed |
| CSIRO Kick-Start (Voucher 1) | $50,000 (matched) | Q2 2026 | Application pending |
| QLD Advance Queensland | $25,000 | Q3 2026 | To be investigated |
| Resource Recovery Industry Development Program | Up to $5M | Q4 2026 | Expression of interest |
| CSIRO Kick-Start (Voucher 2) | $50,000 (matched) | 2027 | Conditional on V1 completion |

## 8. Team

Archers Group Pty Ltd is a Queensland-based construction and engineering consultancy with direct industry experience in civil earthworks, project management, and regulatory compliance.

| Role | Person | Capability |
| --- | --- | --- |
| Founder / Product | Matthew Patterson | Civil construction industry experience, Archers Group director, domain expertise in earthworks compliance |
| Technical Development | AI-assisted development | Rapid prototyping capability demonstrated (MVP built) |
| Industry Advisory | [TBD — target CCF QLD committee member] | Industry credibility and network |
| Government Advisory | [TBD — target former DES/BQCC officer] | Regulatory expertise and government pathway |

## 9. Risks & Mitigations

| Risk | Likelihood | Impact | Mitigation |
| --- | --- | --- | --- |
| Low user adoption | Medium | High | Free platform, direct industry outreach, government endorsement |
| Regulatory change | Low | Medium | Platform designed for flexibility, ongoing monitoring |
| Competitor enters market | Medium | Medium | First-mover advantage, QLD-specific compliance moat, government relationships |
| Data accuracy issues | Medium | High | Official government data sources, user verification, disclaimer framework |
| Government won't partner | Low | Medium | Platform provides value independently; partnership enhances but isn't required |
| Fire ant zone boundaries change | Certain | Low | Monthly data refresh from official source |
| Legal liability for compliance guidance | Medium | High | Clear disclaimers — guidance only, not legal advice. Link to official sources. |

## 10. Impact Metrics

Cut2Fill will track and report:

| Metric | Year 1 Target | Year 3 Target |
| --- | --- | --- |
| Material diverted from landfill | 50,000 m³ | 500,000 m³ |
| Waste levy savings for industry | $3.8M | $38M |
| Truck trips avoided | 4,200 | 42,000 |
| CO₂ emissions avoided (tonnes) | 630 | 6,300 |
| Registered companies | 200 | 2,000 |
| Active listings | 500 | 5,000 |
| Government partnerships | 2 councils | All SEQ councils + DES |

## Appendices

Appendix A: QLD Regulatory Compliance Framework (see 02-Regulatory-Framework.md)

Appendix B: Feasibility Study & CSIRO Grant Application (see 03-Feasibility-Study.md)

Appendix C: SEQ Registered Facilities Data Sources (see 04-Data-Sources.md)

Appendix D: MVP Technical Specification (see 05-MVP-Specification.md)

Appendix E: Cost Intelligence & Material Specification Engine (see 06-Cost-Intelligence-Feature.md)

Appendix F: Material Tracking, Compliance Automation & Long-Term Vision (see 07-Material-Tracking-and-Vision.md)

# Cut2Fill — QLD Regulatory Compliance Framework

Version: 1.0

Date: 23 March 2026

Purpose: Comprehensive reference for all QLD regulations governing the movement, disposal, and reuse of earthworks materials. This document underpins the compliance intelligence layer of the Cut2Fill platform.

## 1. Regulatory Overview

Moving earthworks material in Queensland is governed by six overlapping regulatory frameworks. Non-compliance with any single framework can result in significant penalties, project delays, and remediation costs.

┌─────────────────────────────────────────────────────────┐
│                    MATERIAL MOVEMENT                     │
├──────────┬──────────┬──────────┬──────────┬─────────────┤
│ EP Act   │Biosecur- │ Planning │  Waste   │   Local     │
│ 1994     │ity Act   │ Act 2016 │  Reduc-  │   Govt      │
│          │ 2014     │   + SPP  │  tion &  │   Laws      │
│          │          │          │  Recyc.  │             │
│          │          │          │  Act 2011│             │
├──────────┼──────────┼──────────┼──────────┼─────────────┤
│General   │Fire ant  │Acid      │Waste     │Fill         │
│environ-  │movement  │sulfate   │levy &    │placement    │
│mental    │controls  │soil      │tracking  │approvals    │
│duty      │BIP req.  │ASSMP     │EOW codes │Erosion &    │
│ERA lic.  │Zone      │          │Regulated │sediment     │
│Contam.   │restrict- │          │waste     │control      │
│land      │ions      │          │transport │             │
└──────────┴──────────┴──────────┴──────────┴─────────────┘

## 2. Environmental Protection Act 1994

### 2.1 General Environmental Duty (Section 319)

Applies to: Every person and company in Queensland conducting any activity.

"A person must not carry out any activity that causes, or is likely to cause, environmental harm unless the person takes all reasonable and practicable measures to prevent or minimise the harm."

Relevance to Cut2Fill: Every material exchange facilitated through the platform must not cause environmental harm. The platform's compliance guidance helps users meet this duty.

### 2.2 Environmentally Relevant Activities (ERAs)

ERAs require an Environmental Authority (EA) from DES. The following ERAs are relevant to earthworks material handling:

| ERA | Description | Threshold | Relevance |
| --- | --- | --- | --- |
| ERA 16 | Extractive and screening activities | >5,000t/year extraction | Quarries, sand/gravel pits, mobile screening |
| ERA 53 | Composting and organic material processing | >200t/year | Soil blending with organic matter |
| ERA 54 | Mechanical waste reprocessing | Any scale | Soil screening, crushing, recycling |
| ERA 55 | Other waste treatment | Any scale | Soil treatment, remediation |
| ERA 56 | Regulated waste storage | >various thresholds | Contaminated soil stockpiling |
| ERA 57 | Regulated waste transport | >250kg at a time | Moving contaminated or regulated soil |
| ERA 60 | Waste disposal — landfill | <50,000t/year to >200,000t/year (4 tiers) | All licensed landfills and disposal sites |

Cut2Fill integration: The platform will identify when a material type or activity triggers ERA requirements and alert the user. Registered facilities will display their ERA licence status.

### 2.3 Contaminated Land (Part 8, Divisions 1-6)

Environmental Management Register (EMR):

Lists land where notifiable activities have been or are being conducted

~20 categories of notifiable activities (Schedule 3 of EP Act)

Includes petroleum storage, landfill, mining, chemical manufacturing, etc.

Contaminated Land Register (CLR):

Lists land where contamination has been confirmed through investigation

Higher risk classification than EMR

Soil Disposal Permits (Section 424-425):

Required when moving soil from EMR or CLR listed land

Free to obtain, decided in 10 business days

Cannot be amended once issued — new permit required for changes

Non-compliance: 100 penalty units ($16,692) per offence

Cut2Fill integration:

When a listing address matches an EMR/CLR listed site, flag the requirement for a Soil Disposal Permit

Provide link to DES Soil Disposal Permit application

Display disclaimer that EMR/CLR checks should be independently verified

### 2.4 Key Offence Provisions

| Section | Offence | Penalty |
| --- | --- | --- |
| s.424 | Depositing contaminated soil without permit | 100 penalty units |
| s.425 | Failing to comply with soil disposal permit conditions | 100 penalty units |
| s.430 | Carrying out ERA without EA | 300 penalty units |
| s.437 | Contravening conditions of EA | 300 penalty units |
| s.440ZG | Unlawful deposit of prescribed waste | 4,500 penalty units or 5 years imprisonment |

1 penalty unit = $166.92 (2025-26)

## 3. Biosecurity Act 2014 — Fire Ant Movement Controls

### 3.1 General Biosecurity Obligation (Section 23)

Every person in Queensland has a General Biosecurity Obligation (GBO) to take all reasonable and practical measures to prevent or minimise biosecurity risks. For fire ants, this specifically applies to the movement of soil, mulch, hay, and other carrier materials.

### 3.2 Biosecurity Zones

Fire ant biosecurity zones are declared under the Biosecurity Act 2014 and updated approximately monthly by Biosecurity Queensland (BQCC).

| Zone | Colour | Restrictions |
| --- | --- | --- |
| Zone 1 (Restricted) | Red | Soil/carrier material must not be moved outside Zone 1 without a BIP. Movement within Zone 1 permitted with GBO compliance. |
| Zone 2 (Buffer) | Orange | Soil/carrier material must not be moved outside Zone 2 without a BIP. Movement within Zone 2 permitted with GBO compliance. Movement from Zone 2 into Zone 1 permitted. |
| Outside zones | — | No fire ant movement restrictions, but GBO still applies. |

### 3.3 What Constitutes "Soil" Under Fire Ant Regulations

The regulations apply to soil within the top 1 metre of the ground surface, including:

All types of soil, earth, fill, clay, sand, loam

Soil attached to plants, machinery, or equipment

Material containing soil (e.g., mulch mixed with soil)

### 3.4 Biosecurity Instrument Permits (BIP)

A BIP is required to move carrier materials (including soil) out of a biosecurity zone. BIPs specify:

Treatment requirements (e.g., chemical treatment, visual inspection)

Documentation requirements

Destination restrictions

Reporting obligations

### 3.5 Penalties

| Offence | Individual | Corporation |
| --- | --- | --- |
| Failing to comply with GBO | 750 penalty units ($125,175) | 3,750 penalty units ($625,875) |
| Breaching movement controls | 750 penalty units ($125,175) | 3,750 penalty units ($625,875) |
| Failing to comply with BIP conditions | 300 penalty units ($50,076) | 1,500 penalty units ($250,380) |

### 3.6 Cut2Fill Integration

Zone checking: Every listing is geocoded and checked against current biosecurity zone boundaries.

Match warnings: When "Find Matches" connects listings in different zones, the platform displays:

Warning banner: "Fire ant biosecurity zone crossing — BIP may be required"

Link to BQCC zone checker tool

Link to BIP application process

Zone data: Sourced from QLD Open Data Portal (data.qld.gov.au) in GPKG/SHP format, or queried live from ArcGIS REST API (Layer 65 of Biosecurity MapServer). Refreshed monthly to match BQCC update cycle.

## 4. Acid Sulfate Soil (ASS) Management

### 4.1 Legislative Framework

ASS management in Queensland is governed by six interacting pieces of legislation:

State Planning Policy (SPP) — triggers assessment requirements

Planning Act 2016 — development assessment framework

Environmental Protection Act 1994 — environmental harm provisions

Coastal Protection and Management Act 1995 — coastal zones

Fisheries Act 1994 — waterways protection

Water Act 2000 — groundwater impacts

### 4.2 When ASS Assessment is Required

Assessment is triggered when works involve:

Excavation of ≥100m³ of soil or sediment at or below 5m AHD (Australian Height Datum)

Filling of ≥500m³ at or below 5m AHD

In some LGAs, the trigger level is 10m AHD (more conservative)

Note: AHD thresholds vary by location. Coastal areas and floodplains are highest risk.

### 4.3 Acid Sulfate Soil Management Plan (ASSMP)

When triggered, an ASSMP must be prepared by a suitably qualified person. The ASSMP covers:

Site investigation and testing (chromium reducible sulfur test — SCr)

Treatment requirements (lime dosing rates)

Monitoring plan (pH, water quality)

Disposal/reuse plan for excavated ASS material

Contingency measures

### 4.4 Cut2Fill Integration

Material type "Acid Sulfate Soil" triggers automatic guidance

Tooltip: "ASSMP may be required below 5m AHD (10m AHD in some LGAs). See SPP guidelines."

Listings for ASS material display a prominent compliance badge

Link to QLD ASS guidelines and SPP mapping

## 5. Waste Reduction and Recycling Act 2011

### 5.1 Waste Levy

The waste levy applies to all waste delivered to a levied waste disposal site in the waste levy zone (which includes all of SEQ).

| Year | Levy Rate |
| --- | --- |
| 2024-25 | $105/tonne |
| 2025-26 | $115/tonne |
| 2026-27 | $125/tonne |
| 2027-28 | $145/tonne |

Critical change: As of 1 July 2023, the previous exemption for clean earth was removed. This means:

Clean fill delivered to a landfill now attracts the full waste levy

A single truck load (~15m³ / ~25 tonnes) of clean fill costs $2,875 in levy alone

This creates a powerful economic incentive for reuse via a platform like Cut2Fill

### 5.2 Exempt Waste

Limited exemptions remain:

Waste from natural disasters (declared under Disaster Management Act)

Waste from emergency events

Some specific industrial by-products with EOW codes

### 5.3 End of Waste (EOW) Codes

EOW codes provide a streamlined pathway for declaring that a material is no longer "waste" and can be used as a resource. As of March 2026, 47 EOW codes have been approved.

Construction-relevant EOW codes:

| Code | Material | Relevance |
| --- | --- | --- |
| ENEW07604819 | Recycled aggregates | Crushed concrete, brick, tile |
| ENEW07278517 | Returned concrete | Unused concrete from batching plants |
| Various | Slag, fly ash, by-products | Industrial by-products for construction |

Critical gap: There is NO EOW code for excavated soil or clean fill.

This means excavated soil is legally classified as "waste" even when it is clean, uncontaminated, and suitable for direct reuse. This regulatory gap:

Forces conservative disposal decisions (landfill rather than reuse)

Creates unnecessary levy costs

Increases truck movements and emissions

Contradicts the intent of the QLD Waste Strategy

Cut2Fill's role: The platform can generate data to support the case for developing an EOW code for clean fill/excavated soil. Transaction data showing volumes of material successfully reused, material testing results, and compliance documentation would provide evidence for a DES submission.

### 5.4 Waste Tracking

Regulated waste transport (ERA 57) requires:

Waste tracking certificates for regulated waste

Transport only by licensed carriers

Destination must hold appropriate EA

Non-regulated waste (including clean fill) does not require waste tracking certificates, but the general environmental duty (EP Act s.319) still applies.

### 5.5 Cut2Fill Integration

Display waste levy calculator: "Sending this material to landfill would cost approximately $X in waste levy alone"

Highlight cost savings from reuse vs. disposal

For contaminated soil listings, flag regulated waste transport requirements

Track EOW code status and notify users when relevant codes are approved

## 6. Planning Act 2016 & Local Government Requirements

### 6.1 Fill Placement Approvals

Most SEQ local governments require development approval for fill placement above certain thresholds:

| Council | Trigger for Approval |
| --- | --- |
| Brisbane City Council | Fill >50m³ in most zones; any fill in waterway corridors or flood areas |
| Gold Coast City Council | Fill requiring change to natural ground level >300mm |
| Sunshine Coast Regional Council | Fill >100m³ or depth >300mm |
| Ipswich City Council | Operational works approval for filling |
| Logan City Council | Fill >100m³ in residential zones |
| Moreton Bay Regional Council | Varies by overlay zone |

Note: Thresholds and requirements vary significantly between LGAs and between zone categories within each LGA. Always check the relevant planning scheme.

### 6.2 Erosion and Sediment Control

All fill placement and earthworks must comply with erosion and sediment control requirements. Most councils reference the IECA (International Erosion Control Association) Best Practice Erosion and Sediment Control guidelines.

### 6.3 Cut2Fill Integration

When a listing involves fill placement, display the receiving LGA's fill approval requirements

Link to relevant planning scheme provisions

Remind users to obtain necessary development approvals before accepting material

## 7. National Environment Protection Measure (NEPM)

### 7.1 NEPM for Assessment of Site Contamination

The NEPM provides the national framework for assessing soil contamination, including:

Health Investigation Levels (HILs) for various land uses

Ecological Investigation Levels (EILs)

Health Screening Levels (HSLs) for vapour intrusion

Ecological Screening Levels (ESLs)

### 7.2 Application in QLD

Queensland adopts the NEPM as the technical standard for contamination assessment. The EP Act provides the enforcement mechanism.

Key standards for material movement:

Virgin Excavated Natural Material (VENM): Soil that has not been subject to any notifiable activity and meets background concentrations. Lowest risk category.

Excavated Natural Material (ENM): Soil from non-contaminated sites that may have slightly elevated but non-hazardous concentrations.

Contaminated Soil: Soil exceeding NEPM investigation levels. Requires disposal at licensed facility or treatment.

### 7.3 Testing Requirements by Material Classification

| Classification | Testing Required | Disposal/Reuse Options |
| --- | --- | --- |
| VENM | Visual assessment + site history (no laboratory testing required if provenance is clear) | Unrestricted reuse |
| ENM | Laboratory testing against NEPM criteria | Reuse with conditions |
| CT-1 (Contaminated Tier 1) | Full laboratory suite per NEPM | Licensed facility only |
| CT-2 (Contaminated Tier 2) | Extended laboratory suite | Licensed high-security facility |

### 7.4 Cut2Fill Integration

"Material Tested" badge with classification level (VENM, ENM, CT-1, CT-2)

Material type tooltips reference NEPM classification requirements

Listings for contaminated material display appropriate warnings and disposal requirements

## 8. Chain of Custody & Documentation Requirements

### 8.1 Legal Requirements

There is no single "chain of custody" law in QLD for all material movements. However, documentation requirements arise from multiple sources:

| Requirement | Source | When Required |
| --- | --- | --- |
| Soil Disposal Permit | EP Act s.424 | Soil from EMR/CLR land |
| Waste tracking certificate | EP Regulation | Regulated waste transport |
| BIP documentation | Biosecurity Act | Material crossing fire ant zones |
| ASSMP | SPP + EP Act | ASS material excavation |
| Test results / NATA certificates | NEPM + commercial requirements | Whenever material classification is disputed |
| Transport records | General environmental duty + commercial | All material movements (best practice) |

### 8.2 Best Practice Documentation

For commercial transactions, Cut2Fill will encourage (but not mandate):

Material source declaration (project name, address, site history)

Material classification (testing results if available)

Volume and load records

Destination confirmation

Transport details

Fire ant zone compliance confirmation

### 8.3 Cut2Fill Integration (Phase 2)

Optional compliance documentation generation per transaction

Material passport concept — digital record of material provenance, testing, and movement

Downloadable PDF for site files and audits

Integration with transport records

## 9. Compliance Decision Matrix

This matrix summarises what is required based on material type and circumstances:

| Material Type | Fire Ant Zone Crossing | From EMR/CLR Land | Receiving LGA Approval | Testing Required | Special Permit |
| --- | --- | --- | --- | --- | --- |
| Clean Fill | BIP required | Soil Disposal Permit | Fill placement approval if over threshold | No (but recommended) | No |
| Rock / Rubble | BIP required (if mixed with soil) | Soil Disposal Permit | Fill placement approval | No | No |
| Sand | BIP required | Soil Disposal Permit | Fill placement approval | ASS screening if coastal | No |
| Topsoil | BIP required | Soil Disposal Permit | Fill placement approval | Weed seed cert. recommended | No |
| Concrete / Asphalt | BIP required (if mixed with soil) | Soil Disposal Permit | Fill placement approval | Asbestos clearance | EOW code applies |
| Mixed C&D | BIP required | Soil Disposal Permit | Not for reuse — licensed facility only | Yes — waste classification | ERA 54 or 60 |
| Acid Sulfate Soil | BIP required | Soil Disposal Permit | ASSMP required + LGA approval | Yes — SCr test | ASSMP approval |
| Contaminated Soil | BIP required + additional conditions | Soil Disposal Permit REQUIRED | Not for reuse — licensed facility only | Yes — full NEPM suite | Soil Disposal Permit + waste tracking |

## 10. Regulatory Gap Analysis & Opportunities

### 10.1 Gaps Identified

No EOW code for clean fill — the most commonly moved material type has no streamlined regulatory pathway for reuse

No digital fire ant compliance tool — zone checking is manual via BQCC website

No centralised material movement register — government has no visibility on where material is going

No real-time facility capacity data — landfills and dump sites publish no capacity information

Environmental Authorities register lacks coordinates — the most authoritative facility register has no spatial data

### 10.2 Opportunities for Cut2Fill

| Gap | Cut2Fill Solution | Government Benefit |
| --- | --- | --- |
| No EOW code for clean fill | Platform data demonstrates safe reuse at scale | Evidence base for EOW code development |
| No digital zone compliance | Automated zone checking and warnings | Improved biosecurity compliance |
| No material movement register | Platform tracks all facilitated exchanges | Policy intelligence, waste strategy monitoring |
| No capacity data | Facilities self-report capacity on platform | Proactive capacity planning |
| No spatial EA data | Cut2Fill geocodes and maps all licensed facilities | Public information access |

## References

### Primary Legislation

Environmental Protection Act 1994 (Qld) — https://www.legislation.qld.gov.au/view/html/inforce/current/act-1994-062

Environmental Protection Regulation 2019 (Qld) — https://www.legislation.qld.gov.au/view/whole/html/inforce/current/sl-2019-0155

Biosecurity Act 2014 (Qld) — https://www.legislation.qld.gov.au/view/html/inforce/current/act-2014-007

Waste Reduction and Recycling Act 2011 (Qld) — https://www.legislation.qld.gov.au/view/html/inforce/current/act-2011-031

Planning Act 2016 (Qld) — https://www.legislation.qld.gov.au/view/html/inforce/current/act-2016-025

### Government Resources

Soil Disposal Permits — https://www.qld.gov.au/environment/management/environmental/contaminated-land/permits/disposal-permit

Fire Ant Biosecurity Zones — https://www.fireants.org.au/stop/biosecurity-zones

Fire Ant Movement Controls Guide — https://www.fireants.org.au/stop/movement-controls/movement-controls-guide

End of Waste Codes — https://www.business.qld.gov.au/running-business/environment/waste-management/regulated-waste/eow-codes

ASS Legislation & Policies — https://www.qld.gov.au/environment/land/management/soil/acid-sulfate/legislation-policies

Waste Levy Rates — https://www.qld.gov.au/environment/circular-economy-waste-reduction/disposal-levy/about/levy-rates

NEPM Assessment of Site Contamination — https://www.nepc.gov.au/nepms/assessment-site-contamination

QLD Waste Strategy — https://www.qld.gov.au/environment/circular-economy-waste-reduction/strategy-plans/strategy

EA Public Register — https://apps.des.qld.gov.au/public-register/search/ea.php

### Data Sources

Fire Ant Zone Data (SHP/GPKG) — https://www.data.qld.gov.au/dataset/fire-ant-biosecurity-zones-queensland

QLD Waste Facilities — https://www.data.qld.gov.au/dataset/public-waste-and-recycling-facilities-in-queensland

Environmental Authorities Register — https://www.data.qld.gov.au/dataset/environmental-authorities

Mining Permits ArcGIS REST — https://spatial-gis.information.qld.gov.au/arcgis/rest/services/Economy/MinesPermitsCurrent/MapServer

Key Resource Areas — https://www.data.qld.gov.au/dataset/key-resource-areas-queensland-series

# Cut2Fill — Feasibility Study & CSIRO Kick-Start Grant Application Brief

Version: 1.0

Date: 23 March 2026

Applicant: Archers Group Pty Ltd

## 1. Feasibility Assessment

### 1.1 Technical Feasibility — CONFIRMED

| Component | Feasibility | Evidence |
| --- | --- | --- |
| Interactive mapping platform | Proven | MVP prototype built and functional (Leaflet.js) |
| Fire ant zone overlay | Proven | QLD Open Data provides official zone boundaries in SHP/GPKG/GeoJSON; ArcGIS REST API available for live query |
| Registered facility mapping | Feasible | QLD Waste Facilities CSV (geocoded), EA Register (requires geocoding), Mining Permits (ArcGIS REST with coordinates) |
| Material matching algorithm | Feasible | Logic demonstrated in MVP; spatial + material + date matching achievable |
| Compliance intelligence | Feasible | Regulatory framework fully mapped (see 02-Regulatory-Framework.md); decision tree logic implementable |
| User authentication & data persistence | Standard | PostgreSQL + PostGIS + standard auth patterns |
| Mobile responsiveness | Proven | MVP is responsive; field workers need tablet/phone access |

Key technical risk: The Environmental Authorities register (most authoritative facility dataset) contains text addresses but no coordinates. Geocoding 1,000+ addresses is required. Mitigation: Use Google Maps Geocoding API or Mapbox — cost approximately $5/1,000 addresses. One-time bulk geocoding with monthly incremental updates.

### 1.2 Market Feasibility — STRONG

Demand indicators:

Waste levy escalation creates urgent economic need. At $115/tonne (2025-26), a single subdivision project generating 10,000m³ of clean fill (approximately 17,000 tonnes) faces a potential levy cost of $1.96M if sent to landfill. The incentive to find reuse options is overwhelming and growing annually.

Clean earth levy exemption removal (1 July 2023) is recent. Industry is still adapting to the new cost reality. Many smaller operators are unaware of the exemption removal or don't know their options.

Brisbane 2032 Olympics pipeline. The largest construction program in QLD history is underway, generating and requiring massive volumes of material. Government will need visibility on material flows.

No existing digital solution. Industry currently relies on phone calls, personal networks, and occasional social media posts. The "Dial Before You Dump" phone service has no digital platform.

Government alignment. QLD Waste Management and Resource Recovery Strategy explicitly targets increased C&D waste diversion. The Resource Recovery Industry Development Program funds solutions in this space (up to $10M grants).

Demand validation planned:

Survey of 30 civil earthworks companies in SEQ (Archers Group network)

Interviews with 5 council waste management officers

Letter of support from CCF QLD (Civil Contractors Federation)

Letter of support from at least 2 SEQ councils

### 1.3 Financial Feasibility — VIABLE

Break-even analysis:

| Scenario | Monthly Cost | Revenue Required | Users Required |
| --- | --- | --- | --- |
| MVP (Year 1) | $5,300/month | $0 (funded) | N/A — user acquisition phase |
| Growth (Year 2) | $16,000/month | $15,000/month | 100 premium subscribers @ $50/month + API clients |
| Scale (Year 3) | $27,000/month | $62,500/month | 200 subscribers + transaction fees + govt contracts |

Key financial insight: The platform's value to a single large contractor can be demonstrated in a single transaction. One project that avoids sending 5,000m³ of clean fill to landfill saves approximately $575,000 in waste levy costs. Against that saving, a $50/month platform subscription is effectively zero cost.

### 1.4 Regulatory Feasibility — FAVOURABLE

No regulatory barriers to operating a material exchange platform

Platform facilitates compliance, does not replace or undermine it

Government data sources are publicly available under open licenses (CC-BY 4.0)

Fire ant zone data is published with intent for third-party use

Appropriate disclaimers protect against liability (guidance only, not legal advice)

### 1.5 Organisational Feasibility — ACHIEVABLE

Archers Group capabilities:

Direct construction industry experience and networks

Understanding of QLD earthworks regulatory environment

Demonstrated rapid prototyping capability (MVP built)

Domain acquired (cut2fill)

Existing business entity with ABN, GST registration, ACN

Gaps to fill:

Dedicated product/development resource (CSIRO collaboration can fill R&D component)

Government relationship manager (target: former DES or BQCC staff member)

Industry advisory board (target: 3-5 members from major contractors, council, DES)

## 2. CSIRO Kick-Start Application Brief

### 2.1 Program Alignment

| CSIRO Requirement | Cut2Fill Fit |
| --- | --- |
| Turnover <$10M or trading <3 years | Archers Group meets one of these criteria |
| ACN held | Yes |
| GST registered | Yes |
| Cash matched funding ($10K-$50K) | Archers Group commits $50,000 cash match |
| R&D requiring scientific/technical expertise | Spatial data integration, matching algorithms, compliance decision modelling |
| 2-12 month project duration | 12-month project proposed |
| Aligns with CSIRO capability | CSIRO Data61 (spatial analytics), CSIRO Manufacturing (circular economy), CSIRO Environment (waste and contamination) |

### 2.2 Proposed R&D Project: "Intelligent Material Exchange for Construction Circular Economy"

Project Title: Development of a Spatially-Intelligent Material Exchange Platform for Construction Earthworks in South East Queensland

Duration: 12 months

CSIRO Funding Requested: $50,000

Archers Group Cash Match: $50,000

Total Project Value: $100,000

### 2.3 R&D Challenge Statement

"How can spatial data analytics, regulatory intelligence, and material classification science be combined into a digital platform that optimises the reuse of construction earthworks material while ensuring compliance with QLD's complex, overlapping environmental, biosecurity, and planning regulations?"

This challenge requires expertise beyond the applicant's capability in three areas:

1. Spatial Data Science

Integration of multiple government spatial datasets (fire ant zones, mining permits, waste facilities, KRA overlays) into a unified geospatial engine

Development of a spatial matching algorithm that considers proximity, transport corridors, zone boundaries, and LGA boundaries

Geocoding and validation of 1,000+ facility addresses from the Environmental Authorities register

2. Environmental Science / Contamination Assessment

Development of a material classification decision support model based on NEPM criteria

Risk scoring for material types based on provenance, testing status, and regulatory classification

Assessment of the feasibility and data requirements for developing an End of Waste (EOW) code submission for clean fill/excavated soil

3. Data Analytics / Circular Economy Modelling

Development of metrics for measuring material diversion from landfill

Carbon emission reduction modelling (avoided truck trips, avoided virgin material extraction)

Economic modelling of waste levy savings and industry benefit

Regional material balance analysis (supply vs demand by LGA)

### 2.4 Proposed Work Packages

| WP | Title | CSIRO Contribution | Archers Contribution | Duration |
| --- | --- | --- | --- | --- |
| WP1 | Geospatial Data Integration & Validation | Spatial data science expertise, geocoding methodology, data quality framework | Platform development, data sourcing, field validation | Months 1-4 |
| WP2 | Material Classification Decision Model | Environmental science expertise, NEPM interpretation, risk assessment methodology | Industry input on practical workflows, user interface design | Months 3-7 |
| WP3 | Matching Algorithm & Compliance Engine | Algorithm design, spatial optimisation, constraint modelling (zone boundaries, transport corridors) | Platform integration, testing with real industry scenarios | Months 5-9 |
| WP4 | Impact Measurement & EOW Feasibility | Circular economy modelling, carbon accounting methodology, policy analysis | Data collection, industry engagement, stakeholder liaison | Months 8-12 |

### 2.5 Expected Outcomes

| Outcome | Measure | Timeframe |
| --- | --- | --- |
| Functional platform with integrated government spatial data | All SEQ registered facilities mapped with verified coordinates | Month 6 |
| Material classification decision support tool | Validated against 50 real material exchange scenarios | Month 9 |
| Spatial matching algorithm with compliance checking | Demonstrated zone-crossing detection accuracy >99% | Month 9 |
| Circular economy impact model | Peer-reviewable methodology for material diversion metrics | Month 12 |
| EOW code feasibility report | Assessment of data requirements and regulatory pathway | Month 12 |
| Pilot with industry | 50 registered companies, 10 facilitated exchanges | Month 12 |

### 2.6 Why CSIRO?

The R&D challenge sits at the intersection of:

Data61 — spatial data integration, algorithm development, geospatial analytics

CSIRO Environment — contaminated land science, waste management, regulatory frameworks

CSIRO Manufacturing — circular economy research, industrial ecology, material flows

CSIRO's existing research programs in Circular Economy and Sustainable Materials Management directly align with this project. CSIRO has published extensively on construction waste diversion and circular economy metrics.

### 2.7 Commercialisation Pathway

| Phase | Milestone | Revenue |
| --- | --- | --- |
| R&D (Year 1) | Platform development, data integration, pilot | $0 (grant-funded) |
| Market entry (Year 2) | 200 companies, premium features launch | $180K ARR |
| Growth (Year 3) | Government contracts, state-wide expansion | $750K ARR |
| Scale (Year 4-5) | National platform, multiple state jurisdictions | $2-5M ARR |

Exit / scale pathways:

Acquisition by government (platform becomes public infrastructure)

Acquisition by construction technology company (e.g., Procore, Aconex)

Partnership/licensing with waste management companies (Cleanaway, Veolia, Remondis)

SaaS growth to national platform (each state has equivalent regulations)

### 2.8 National Scalability

Every Australian state has:

State-specific contaminated land registers

Waste levies (NSW, VIC, QLD, SA, WA)

Planning requirements for fill placement

Biosecurity obligations (fire ants specific to QLD but other pests nationally)

Construction material movement challenges

The QLD platform serves as the proof of concept. The regulatory intelligence layer is modular — state-specific rules can be added as configuration, not code rewrites.

## 3. Risk Assessment

### 3.1 Technical Risks

| Risk | Probability | Impact | Mitigation |
| --- | --- | --- | --- |
| Government data quality issues | High | Medium | Cross-reference multiple sources; build user-reported correction mechanism |
| Geocoding errors for EA register | Medium | Medium | Manual validation of key facilities; use multiple geocoding services |
| Fire ant zone boundary precision | Low | High | Use official data source; monthly refresh; disclaimer on approximate boundary display |
| Matching algorithm produces poor matches | Medium | Medium | Human review loop in MVP; iterate algorithm based on user feedback |
| Platform scalability under load | Low | Low | Cloud hosting with auto-scaling; CDN for static assets |

### 3.2 Market Risks

| Risk | Probability | Impact | Mitigation |
| --- | --- | --- | --- |
| Industry slow to adopt digital tools | Medium | High | Free tier removes cost barrier; direct outreach via industry bodies; demonstrate ROI per transaction |
| Government unwilling to partner | Low | Medium | Platform provides value independently; partnership enhances but isn't required |
| Competitor launches similar platform | Medium | Medium | First-mover advantage; compliance depth as moat; government relationships |
| Users post inaccurate listings | High | Medium | Verification process; user reputation system; disclaimers |

### 3.3 Regulatory Risks

| Risk | Probability | Impact | Mitigation |
| --- | --- | --- | --- |
| Liability for compliance guidance | Medium | High | Clear disclaimers (guidance only, not legal advice); link to official sources; professional indemnity insurance |
| Regulation changes invalidate guidance | Medium | Medium | Modular compliance rules; monitoring of legislative changes; automated alerts |
| Platform perceived as facilitating non-compliance | Low | Very High | Compliance-first design; government partnership; audit trail; refuse listings for obviously non-compliant material |

## 4. Letters of Support (To Be Obtained)

The following letters of support will strengthen the CSIRO application:

| Organisation | Contact | Purpose |
| --- | --- | --- |
| Civil Contractors Federation QLD | CEO/GM | Industry body endorsement |
| Brisbane City Council (Waste & Resource Recovery) | Manager | LGA support, data access |
| Logan City Council | Waste officer | LGA support, pilot partnership |
| Biosecurity Queensland (BQCC) | Director | Fire ant data partnership |
| QLD Department of Environment & Science | Waste policy officer | Regulatory alignment |
| 2-3 civil earthworks companies | Directors | End-user demand validation |

## 5. Budget Breakdown (CSIRO Application)

### 5.1 CSIRO Contribution ($50,000)

| Item | Amount | Detail |
| --- | --- | --- |
| Spatial data scientist (0.2 FTE, 12 months) | $25,000 | Geospatial integration, geocoding, algorithm design |
| Environmental scientist (0.1 FTE, 6 months) | $12,500 | Material classification model, NEPM, EOW feasibility |
| Circular economy analyst (0.1 FTE, 4 months) | $7,500 | Impact metrics, carbon accounting, policy analysis |
| Computing / data infrastructure | $5,000 | Cloud resources, data processing |

### 5.2 Archers Group Cash Match ($50,000)

| Item | Amount | Detail |
| --- | --- | --- |
| Platform development | $25,000 | Frontend, backend, database, hosting |
| Data acquisition & processing | $5,000 | Geocoding API costs, data licensing (if any) |
| Industry engagement & validation | $8,000 | Travel, meetings, pilot support |
| Project management | $7,000 | Coordination, reporting, milestone tracking |
| Legal & IP | $5,000 | Terms of service, disclaimers, IP protection |

## 6. Timeline

2026 Q2 (Apr-Jun)     CSIRO EOI submission and scoping
                       ├─ Submit Expression of Interest
                       ├─ Scope project with CSIRO researcher
                       └─ Secure letters of support

2026 Q3 (Jul-Sep)     WP1: Geospatial Data Integration
                       ├─ Integrate fire ant zone data (official source)
                       ├─ Geocode EA register facilities
                       ├─ Map quarries from Mining Permits REST API
                       ├─ Load KRA overlays
                       └─ Data quality validation

2026 Q3-Q4             WP2: Material Classification Model
                       ├─ Map NEPM criteria to decision logic
                       ├─ Build classification guidance engine
                       ├─ Validate with environmental consultants
                       └─ Integrate into platform

2026 Q4-2027 Q1        WP3: Matching Algorithm & Compliance
                       ├─ Spatial matching with zone awareness
                       ├─ Compliance decision engine
                       ├─ Transport corridor optimisation
                       └─ Beta testing with industry partners

2027 Q1-Q2             WP4: Impact & EOW Feasibility
                       ├─ Circular economy metrics framework
                       ├─ Carbon accounting methodology
                       ├─ EOW code feasibility assessment
                       ├─ Pilot results analysis
                       └─ Final report and publication

## 7. Intellectual Property

| IP Component | Owner | Notes |
| --- | --- | --- |
| Platform code | Archers Group | Developed independently and with CSIRO collaboration |
| Matching algorithm | Joint — negotiable with CSIRO | Core R&D output |
| Material classification model | Joint — negotiable with CSIRO | Based on public regulatory frameworks |
| Impact measurement methodology | Joint — negotiable with CSIRO | Publishable/shareable for industry benefit |
| Government data integrations | N/A — public data | CC-BY 4.0 licensed, no IP restrictions |
| Domain and brand (Cut2Fill) | Archers Group | Trademark to be registered |

CSIRO Kick-Start IP note: Standard CSIRO Kick-Start terms provide for IP to be owned by the business with CSIRO retaining rights to publish and use for further research. Specific terms negotiated per project.

# Cut2Fill — SEQ Data Sources & Integration Strategy

Version: 1.0

Date: 23 March 2026

## 1. Data Source Inventory

### 1.1 Fire Ant Biosecurity Zones

| Field | Detail |
| --- | --- |
| Source | QLD Open Data Portal / Biosecurity Queensland |
| URL | https://www.data.qld.gov.au/dataset/fire-ant-biosecurity-zones-queensland |
| Formats | SHP, TAB, FGDB, KMZ, GPKG |
| License | CC-BY 4.0 |
| Update frequency | Monthly (non-regular schedule) |
| Last updated | 22 March 2026 |
| Contains | Zone 1 and Zone 2 polygon boundaries |
| Live API | ArcGIS REST: `https://spatial-gis.information.qld.gov.au/arcgis/rest/services/Farming/Biosecurity/MapServer/65` |
| Integration method | Download GPKG for initial load; query ArcGIS REST for live updates. Convert to GeoJSON for Leaflet overlay. |
| Priority | CRITICAL — core compliance feature |

### 1.2 QLD Public Waste & Recycling Facilities

| Field | Detail |
| --- | --- |
| Source | QLD Department of Environment and Science |
| URL | https://www.data.qld.gov.au/dataset/public-waste-and-recycling-facilities-in-queensland |
| Direct CSV | https://data.des.qld.gov.au/__data/assets/file/0021/84351/waste-facilities.csv |
| CKAN API | `https://www.data.qld.gov.au/api/3/action/datastore_search?resource_id=620c726c-646a-4aef-999c-45f9e26cf7d2` |
| Fields | Name, Phone, Website, LGA, Address, Suburb, Postcode, Facility Type, Latitude, Longitude |
| Facility types | Landfills, transfer stations, metal recyclers, C&D recyclers, organic processors, battery recyclers, tyre recyclers, paint recyclers |
| License | CC-BY 4.0 |
| Last updated | October 2018 (STALE — needs supplementation) |
| Integration method | CKAN datastore API with filtering. Lat/lng available for direct map plotting. |
| Priority | HIGH — provides geocoded facility base layer |
| Caveat | Data is 7+ years old. Must cross-reference with EA register for currency. |

### 1.3 Environmental Authorities Register

| Field | Detail |
| --- | --- |
| Source | QLD Department of Environment and Science |
| URL | https://www.data.qld.gov.au/dataset/environmental-authorities |
| Web search | https://apps.des.qld.gov.au/public-register/search/ea.php |
| Fields | Permit Reference, Permit Type, Permit Holder, Effective Date, Status, Condition Type, Industry, Activities, Locations (TEXT ONLY) |
| Formats | XLSX, CSV, TSV, JSON, XML |
| License | CC-BY 4.0 |
| Last updated | 20 March 2026 (CURRENT) |
| Integration method | Download via CKAN API. Filter by ERA codes (16, 53, 54, 55, 56, 57, 60). GEOCODE addresses via Google/Mapbox API to obtain coordinates. |
| Priority | HIGH — authoritative facility register, but requires geocoding |
| Key ERA filters | ERA 16 (quarries), ERA 53 (composting), ERA 54 (waste reprocessing), ERA 55 (waste treatment), ERA 60 (landfill) |

### 1.4 Mining Permits (Quarries)

| Field | Detail |
| --- | --- |
| Source | QLD Department of Resources / Geological Survey |
| ArcGIS REST | `https://spatial-gis.information.qld.gov.au/arcgis/rest/services/Economy/MinesPermitsCurrent/MapServer` |
| Key layers | Layer 38-46 (Mining Leases — granted and applications) |
| Fields | Permit number, holder, status, area, commodity, polygon geometry |
| Format | ArcGIS REST API — JSON/GeoJSON query |
| Integration method | Query by bounding box (SEQ extent) and commodity type (extractive materials). Returns polygon boundaries. |
| Priority | MEDIUM — quarries are important but less dynamic than waste facilities |
| Example query | `/MapServer/42/query?where=1=1&geometry={"xmin":151.5,"ymin":-28.5,"xmax":153.6,"ymax":-26.0}&geometryType=esriGeometryEnvelope&outFields=*&f=json` |

### 1.5 Key Resource Areas (KRA)

| Field | Detail |
| --- | --- |
| Source | QLD Department of Resources |
| URL | https://www.data.qld.gov.au/dataset/key-resource-areas-queensland-series |
| Formats | SHP (4 layers: Resource/Processing Areas, Haulage Corridors, Transport Routes, Separation Areas) |
| ArcGIS REST | `https://spatial-gis.information.qld.gov.au/arcgis/rest/services/GeoscientificInformation/MiningResources/MapServer` |
| WMS | `https://spatial-gis.information.qld.gov.au/arcgis/services/GeoscientificInformation/MiningResources/MapServer/WMSServer` |
| Integration method | Load SHP/GeoJSON as static overlay. KRA data changes infrequently. |
| Priority | LOW for MVP — useful for professional/government audience |

### 1.6 Brisbane City Council Transfer Stations

| Field | Detail |
| --- | --- |
| Source | Brisbane City Council Open Data |
| URL | https://data.brisbane.qld.gov.au/explore/dataset/waste-transfer-stations/information/ |
| Formats | CSV, JSON, Excel, GeoJSON |
| Fields | Name, Address, Opening hours, Latitude, Longitude |
| Integration method | Direct GeoJSON fetch. Small dataset (4 facilities). |
| Priority | LOW — already included in statewide waste facilities CSV |

### 1.7 Acid Sulfate Soil Mapping

| Field | Detail |
| --- | --- |
| Source | QLD Department of Environment and Science |
| URL | https://www.publications.qld.gov.au/dataset/acid-sulfate-soil-guidelines |
| Spatial data | Available via QSpatial (https://qldspatial.information.qld.gov.au/) — search "acid sulfate" |
| Format | Raster/vector layers showing ASS probability |
| Integration method | WMS overlay for visual reference. Not required for MVP but valuable for Phase 2. |
| Priority | LOW for MVP — tooltip guidance sufficient initially |

### 1.8 EMR/CLR Registers

| Field | Detail |
| --- | --- |
| Source | QLD Department of Environment and Science |
| Search tool | https://www.qld.gov.au/environment/management/environmental/contaminated-land/registers/search |
| Format | Web search only — no bulk download or API available |
| Integration method | Cannot integrate directly. Users must independently check EMR/CLR status. Platform provides link and guidance. |
| Priority | N/A for direct integration — provide guidance and links |

## 2. Integration Architecture

### 2.1 Data Pipeline (Phase 2 — Backend)

┌──────────────────────────────────────────────────────────────┐
│                    DATA SOURCES (External)                     │
├────────────┬────────────┬────────────┬────────────┬──────────┤
│ Fire Ant   │ Waste      │ EA         │ Mining     │ KRA      │
│ Zones      │ Facilities │ Register   │ Permits    │          │
│ (ArcGIS)   │ (CKAN CSV) │ (CKAN XLSX)│ (ArcGIS)   │ (SHP)    │
└─────┬──────┴─────┬──────┴─────┬──────┴─────┬──────┴────┬─────┘
      │            │            │            │           │
      ▼            ▼            ▼            ▼           ▼
┌─────────────────────────────────────────────────────────────┐
│              ETL / DATA PROCESSING LAYER                      │
│  ┌─────────┐  ┌──────────┐  ┌───────────┐  ┌────────────┐  │
│  │ GeoJSON │  │ CSV      │  │ Geocoder  │  │ ArcGIS     │  │
│  │ Convert │  │ Parser   │  │ (Mapbox / │  │ REST Query │  │
│  │         │  │          │  │  Google)  │  │            │  │
│  └────┬────┘  └────┬─────┘  └─────┬─────┘  └─────┬──────┘  │
│       │            │              │               │          │
│       ▼            ▼              ▼               ▼          │
│  ┌──────────────────────────────────────────────────────┐   │
│  │         PostgreSQL + PostGIS Database                 │   │
│  │  ┌──────────────┐  ┌─────────────┐  ┌────────────┐  │   │
│  │  │ facilities   │  │ zones       │  │ listings   │  │   │
│  │  │ (points)     │  │ (polygons)  │  │ (user data)│  │   │
│  │  └──────────────┘  └─────────────┘  └────────────┘  │   │
│  └──────────────────────────────────────────────────────┘   │
└─────────────────────────────────────────────────────────────┘
                              │
                              ▼
┌─────────────────────────────────────────────────────────────┐
│                    REST API (FastAPI/Node.js)                 │
│  GET /facilities?type=quarry&bounds=...                      │
│  GET /zones/fire-ant?point=lat,lng                           │
│  GET /listings?material=clean-fill&within=50km               │
│  POST /listings                                              │
│  GET /match/{listingId}                                      │
└─────────────────────────────────────────────────────────────┘
                              │
                              ▼
┌─────────────────────────────────────────────────────────────┐
│                    FRONTEND (Leaflet.js)                      │
│  Map layers: listings, facilities, zones, KRA                │
│  Filters: material, type, pricing, delivery, volume          │
│  Matching: spatial + material + date + zone-aware            │
│  Compliance: tooltips, badges, warnings, decision support    │
└─────────────────────────────────────────────────────────────┘

### 2.2 MVP Data Strategy (No Backend)

For the MVP (static frontend), data is embedded directly:

| Data | Method | Refresh Process |
| --- | --- | --- |
| Fire ant zones | Simplified polygon coordinates in app.js | Manual update from QLD Open Data monthly |
| Sample listings | JSON array in app.js | Manual — sample data only |
| Waste facilities | JSON array in app.js (curated SEQ subset) | Manual from CSV download |
| Material tooltips | Static object in app.js | Manual from regulatory review |

### 2.3 Phase 2 Data Refresh Schedule

| Dataset | Refresh | Method |
| --- | --- | --- |
| Fire ant zones | Monthly | Automated download from QLD Open Data; convert to GeoJSON; deploy |
| EA Register | Monthly | Automated CKAN download; diff against existing; geocode new entries |
| Waste facilities | Quarterly | Check for CSV updates; cross-reference EA register |
| Mining permits | Quarterly | ArcGIS REST query; update quarry layer |
| KRA | Annually | SHP download; unlikely to change frequently |
| User listings | Real-time | User-submitted via platform |

## 3. Known Facility Data (SEQ — For MVP)

### 3.1 Major Landfills

| Facility | Operator | Suburb | Lat | Lng | ERA |
| --- | --- | --- | --- | --- | --- |
| Nudgee Transfer Station | BCC | Nudgee | -27.3711 | 153.0871 | 60 |
| Swanbank Landfill | Remondis | Swanbank | -27.6833 | 152.8500 | 60 |
| Willawong Resource Recovery | Cleanaway | Willawong | -27.5889 | 152.9778 | 60 |
| Reedy Creek Refuse | GCCC | Reedy Creek | -28.1100 | 153.3900 | 60 |
| Caloundra Landfill | SCRC | Caloundra West | -26.7933 | 153.1253 | 60 |
| Ti-Tree Bioenergy | MBRC | Dakabin | -27.2280 | 152.9890 | 60 |
| Staplyton Landfill | Cleanaway | Staplyton | -27.7500 | 153.2700 | 60 |
| Riverview Recycling | Ipswich CC | Riverview | -27.6100 | 152.7800 | 60 |

### 3.2 Major Quarries / Extractive Sites

| Facility | Operator | Suburb | Lat | Lng | Material |
| --- | --- | --- | --- | --- | --- |
| Mt Coot-tha Quarry | Boral | Mt Coot-tha | -27.4850 | 152.9550 | Hard rock |
| Pine Rivers Quarry | Boral | Petrie | -27.2600 | 152.9800 | Hard rock |
| Ormeau Quarry | Hanson | Ormeau | -27.7700 | 153.2500 | Sand, gravel |
| Narangba Quarry | Boral | Narangba | -27.2000 | 152.9600 | Hard rock |
| Purga Quarry | Wagners | Purga | -27.6800 | 152.7200 | Hard rock |
| Wacol Quarry | Various | Wacol | -27.6000 | 152.9300 | Sand |
| Gold Coast Quarry | Boral | Reedy Creek | -28.1200 | 153.3600 | Hard rock |
| Caboolture Quarry | Boral | Caboolture | -27.0500 | 152.9200 | Hard rock |

### 3.3 Licensed Soil Treatment / Recycling

| Facility | Operator | Suburb | Lat | Lng | Capability |
| --- | --- | --- | --- | --- | --- |
| Hi-Quality PFAS Treatment | HiQ Group | Yatala | -27.7300 | 153.2200 | PFAS soil treatment (350,000t/yr) |
| ResourceCo | ResourceCo | Hemmant | -27.4500 | 153.1200 | C&D waste, soil recycling |
| Rino Recycling | Rino | Pinkenba | -27.4200 | 153.1100 | Soil, sand, aggregate recycling |
| Barry's Recycling (Yatala) | Barry's | Yatala | -27.7400 | 153.2300 | Concrete, soil, aggregate |
| Barry's Recycling (Redland Bay) | Barry's | Redland Bay | -27.6300 | 153.3000 | Concrete, soil, aggregate |
| EnviroSoil Solutions | EnviroSoil | Wacol | -27.6400 | 152.9200 | Contaminated soil treatment |
| Boral Recycling | Boral | Various | Various | Various | VENM, CT-1 soils, clean fill |

## 4. API Endpoint Examples

### Fire Ant Zones (ArcGIS REST)

Get all zone boundaries as GeoJSON:

GET https://spatial-gis.information.qld.gov.au/arcgis/rest/services/Farming/Biosecurity/MapServer/65/query?where=1%3D1&outFields=*&f=geojson

Check if a point is in a fire ant zone:

GET https://spatial-gis.information.qld.gov.au/arcgis/rest/services/Farming/Biosecurity/MapServer/65/query?geometry=153.02,-27.47&geometryType=esriGeometryPoint&spatialRel=esriSpatialRelIntersects&outFields=*&f=json

### QLD Waste Facilities (CKAN Datastore)

Get all SEQ facilities:

GET https://www.data.qld.gov.au/api/3/action/datastore_search?resource_id=620c726c-646a-4aef-999c-45f9e26cf7d2&limit=500

Filter by LGA:

GET https://www.data.qld.gov.au/api/3/action/datastore_search?resource_id=620c726c-646a-4aef-999c-45f9e26cf7d2&filters={"LGA":"BRISBANE"}

### Environmental Authorities (CKAN Datastore)

Search for ERA 60 (landfill) facilities:

GET https://www.data.qld.gov.au/api/3/action/datastore_search?resource_id=<resource_id>&q=ERA%2060

### Mining Permits (ArcGIS REST)

Query granted mining leases in SEQ bounding box:

GET https://spatial-gis.information.qld.gov.au/arcgis/rest/services/Economy/MinesPermitsCurrent/MapServer/42/query?where=1%3D1&geometry=%7B%22xmin%22%3A151.5%2C%22ymin%22%3A-28.5%2C%22xmax%22%3A153.6%2C%22ymax%22%3A-26.0%7D&geometryType=esriGeometryEnvelope&spatialRel=esriSpatialRelIntersects&outFields=*&returnGeometry=true&f=json

# Cut2Fill — MVP Technical Specification

Version: 1.0

Date: 23 March 2026

## 1. MVP Scope

### 1.1 In Scope

| Feature | Description |
| --- | --- |
| Cut2Fill branding | Logo, tagline, page title |
| Interactive map | Leaflet.js with SEQ region, dark theme |
| Listing types | Fill Available, Fill Wanted, Public Tip/Transfer Station, Private Dump Site |
| Material types (8) | Clean Fill, Rock/Rubble, Sand, Topsoil, Concrete/Asphalt, Mixed C&D, Acid Sulfate Soil, Contaminated Soil |
| Material compliance tooltips | QLD-specific regulatory guidance per material type |
| Pricing | Free / Quote Required (two-tier) |
| Delivery/Pickup | Separate checkboxes with icon badges |
| Date availability badges | Available Now, Starting Soon, Expiring Soon |
| Fire ant zone overlays | Zone 1 (red), Zone 2 (orange) — toggle on/off |
| Zone crossing warnings | Warning banner when matching across zones |
| BQCC zone checker link | Direct link to official zone checking tool |
| Registered facilities | Curated SEQ quarries, tips, treatment facilities as map markers |
| Matching engine | Material + zone-aware matching with visual connection lines |
| Filters | Type, material, pricing, status, delivery, volume range, search |
| Listings view | Card grid with badges, sorting |
| Dashboard | Material flow stats, regional bars, material chart, recent matches |
| Post listing form | Full form with material tooltips, pricing radios, delivery/pickup checkboxes |
| Minimum volume | 100m³ enforced in form |

### 1.2 Out of Scope (Phase 2)

| Feature | Reason |
| --- | --- |
| User authentication | Requires backend |
| Data persistence | Requires database |
| Real-time notifications | Requires backend + email/SMS service |
| Live government data integration | Requires ETL pipeline |
| Transport cost calculator | Requires routing API and cost model |
| Document upload (geotech reports) | Requires file storage |
| Material specification matching (MRTS) | Requires specification database |
| Compliance documentation generation | Requires templating engine |
| Mobile app | Web-first approach sufficient for MVP |

## 2. Minimum Volume Enforcement

The form enforces a 100m³ minimum to maintain professional positioning:

// In form validation
document.getElementById('formVolume').min = 100;
// Placeholder updated to reflect minimum
// "e.g. 500 (minimum 100m³)"

Sample data updated to remove listings below 100m³ threshold.

## 3. Registered Facility Layer

Facilities are displayed as a distinct marker layer (different from user-posted listings):

| Facility Type | Icon | Colour | Source |
| --- | --- | --- | --- |
| Quarry | `fa-gem` | Grey (#9ca3af) | QLD Mining Permits |
| Landfill | `fa-dumpster` | Brown (#92400e) | QLD Waste Facilities |
| Transfer Station | `fa-recycle` | Blue (#3b82f6) | QLD Waste Facilities |
| Soil Treatment | `fa-flask` | Purple (#8b5cf6) | EA Register |
| C&D Recycler | `fa-cogs` | Teal (#14b8a6) | EA Register |
| PFAS Treatment | `fa-biohazard` | Red (#ef4444) | EA Register |

Facilities are always visible on the map (not affected by listing filters) with a separate toggle control.

## 4. File Structure

C:/material-exchange/          (development)
├── index.html                 Main application
├── styles.css                 Stylesheet
├── app.js                     Application logic + sample data
└── data/                      (Phase 2 — static data files)
    ├── facilities.json        Curated SEQ facilities
    ├── fire-ant-zones.json    Simplified zone polygons
    └── quarries.json          SEQ quarry locations

Cut2Fill/                      (OneDrive — project files)
├── index.html                 Copy of MVP
├── styles.css                 Copy of MVP
├── app.js                     Copy of MVP
├── mvp/                       (future: production build)
└── docs/
    ├── 01-Business-Plan.md
    ├── 02-Regulatory-Framework.md
    ├── 03-Feasibility-Study.md
    ├── 04-Data-Sources.md
    └── 05-MVP-Specification.md

## 5. Deployment

### MVP (Current)

Static files served via npx http-server on port 8070

No backend required

All data embedded in app.js

### Phase 2

Frontend: Static hosting (Netlify, Vercel, or CloudFront)

Backend: FastAPI or Node.js on AWS/GCP

Database: PostgreSQL + PostGIS on managed service

Domain: cut2fill.com.au → DNS configured to hosting

## 6. Browser Support

| Browser | Support |
| --- | --- |
| Chrome (desktop + mobile) | Full |
| Safari (desktop + iOS) | Full |
| Firefox | Full |
| Edge | Full |
| IE11 | Not supported |

## 7. Performance Targets

| Metric | Target |
| --- | --- |
| Initial page load | <2 seconds on 4G |
| Map interaction (zoom/pan) | 60fps |
| Filter application | <100ms |
| Marker rendering (25 listings) | <50ms |
| Marker rendering (500 listings — Phase 2) | <200ms with clustering |

# Cut2Fill — Cost Intelligence & Material Specification Engine

Version: 1.0

Date: 23 March 2026

Status: Feature specification (Phase 2-3)

## 1. The Problem

Finding material is only half the equation. A project manager needs to know:

What will this actually cost me? — Free fill 50km away may cost more than paid fill 5km away once you factor in trucks, fuel, hours, and traffic

Will this material meet my spec? — MRTS05 Type 2.1 material has specific grading, plasticity, and strength requirements. A listing that says "clean fill" tells you nothing about compliance with design specifications

What's my risk? — Wet material costs more to handle, contaminated material has legal liability, untested material may fail on placement

Can I win work with this? — Demonstrating material reuse, reduced truck movements, and lower carbon emissions is increasingly a tender evaluation criterion

No existing platform addresses any of these. Cut2Fill's cost intelligence engine turns the platform from a listing board into a decision support tool that construction professionals trust with real money.

## 2. True Cost Calculator

### 2.1 Cost Factors

The "free" material myth: a listing marked "Free" only means the material itself has no purchase price. The true cost includes:

| Cost Factor | Variables | Data Source |
| --- | --- | --- |
| Material cost | Free or quoted price per m³ | Listing data |
| Truck hire | Rate per hour (typically $140-180/hr for 6-wheeler, $200-250/hr for truck & dog) | Configurable rates |
| Cartage distance | Driving distance in km (not straight line) | Routing API (Google/Mapbox) |
| Travel time | Minutes per trip including loading/unloading | Routing API + configurable load/unload times |
| Fuel consumption | Litres per km by truck type | Standard consumption tables |
| Fuel price | Current diesel price per litre | AIP or manual update |
| Number of loads | Total volume ÷ truck capacity (m³ per load by truck type) | Calculation |
| Loading method | Self-load vs machine-loaded (affects time per load) | Listing data |
| Time of day | Day rate vs night rate (overtime/penalty rates, noise restrictions) | User input |
| Material condition | Wet material = heavier = fewer m³ per load, harder to handle | Listing flag |
| Waste levy (if landfill) | $115/t (2025-26), escalating annually | QLD levy schedule |
| Permit costs | BIP application, Soil Disposal Permit, ASSMP preparation | Regulatory schedule |

### 2.2 Cost Comparison View

When a user clicks "Find Matches" or browses listings, the platform calculates and displays:

┌─────────────────────────────────────────────────────────────┐
│  Your project: Springfield Rise Estate                       │
│  Need: 15,000 m³ Clean Fill                                 │
├─────────────────────────────────────────────────────────────┤
│                                                              │
│  OPTION A: BrisConnect Civil — South Brisbane (32km)        │
│  ├─ Material: FREE                                          │
│  ├─ Cartage: 1,250 loads × $185/load = $231,250            │
│  ├─ Fuel: 1,250 × 64km × 0.45L/km × $1.85 = $66,600      │
│  ├─ Zone crossing: YES — BIP may be required                │
│  ├─ TOTAL ESTIMATED: $297,850 ($19.86/m³)                   │
│  └─ Confidence: ●●●○○ MEDIUM (untested, zone crossing)     │
│                                                              │
│  OPTION B: Urban Dig Co — West End (28km)                   │
│  ├─ Material: FREE                                          │
│  ├─ Cartage: 500 loads × $185/load = $92,500               │
│  ├─ Fuel: 500 × 56km × 0.45L/km × $1.85 = $23,310        │
│  ├─ Zone crossing: NO                                       │
│  ├─ Material Tested: YES ✓                                  │
│  ├─ TOTAL ESTIMATED: $115,810 ($19.30/m³)                   │
│  └─ Confidence: ●●●●○ HIGH (tested, same zone, 6,000m³)   │
│                                                              │
│  OPTION C: Landfill disposal (Swanbank, 18km)               │
│  ├─ Waste levy: 15,000m³ × 1.7t/m³ × $115/t = $2,932,500  │
│  ├─ Cartage: 1,250 loads × $165/load = $206,250            │
│  ├─ Gate fee: est. $15/t × 25,500t = $382,500              │
│  ├─ TOTAL ESTIMATED: $3,521,250 ($234.75/m³)                │
│  └─ YOU SAVE: $3,405,440 by choosing Option B               │
│                                                              │
│  ⚡ SUSTAINABILITY: Option B saves 62,500 truck-km,          │
│     28.1 tonnes CO₂, and diverts 15,000m³ from landfill     │
│                                                              │
└─────────────────────────────────────────────────────────────┘

### 2.3 Truck Types & Capacities

| Truck Type | Capacity (m³) | Hourly Rate (est.) | L/km (loaded) |
| --- | --- | --- | --- |
| 6-wheeler tipper | 5-6 m³ | $140-160/hr | 0.35 |
| 8-wheeler tipper | 8-10 m³ | $160-180/hr | 0.40 |
| Truck & dog | 12-14 m³ | $200-250/hr | 0.45 |
| Semi-tipper | 16-20 m³ | $220-280/hr | 0.50 |
| B-double | 28-34 m³ | $280-350/hr | 0.55 |

### 2.4 Wet vs Dry Material

Wet material impacts cost in three ways:

Heavier — fewer m³ per load before hitting legal weight limits

Harder to handle — slippery, compaction issues, may require drying time

Specification non-compliance — many fill specifications have moisture content limits

| Condition | Bulk density (t/m³) | Load reduction |
| --- | --- | --- |
| Dry fill | 1.5-1.7 | Baseline |
| Moist fill | 1.7-1.9 | 10-15% fewer m³/load |
| Wet/saturated fill | 1.9-2.2 | 20-30% fewer m³/load |

Platform flag: When a listing is marked "wet material," the cost calculator adjusts load counts and displays a warning about potential specification issues.

### 2.5 Night Work Premium

Many sites restrict hours (typically 6am-6pm). If material must be moved outside standard hours:

| Time | Rate Factor | Notes |
| --- | --- | --- |
| Standard (6am-6pm Mon-Fri) | 1.0x | Base rate |
| Saturday | 1.3-1.5x | Penalty rates apply |
| Sunday | 1.5-2.0x | Higher penalties |
| Night (6pm-6am) | 1.5-1.75x | Noise restrictions may apply — check council |

## 3. Material Specification Matching

### 3.1 The Specification Problem

A listing that says "Clean Fill" is meaningless to an engineer who needs material complying with MRTS05 — Earthworks (Queensland Transport and Main Roads Technical Specification). The spec defines multiple material classes with strict requirements.

### 3.2 MRTS05 Material Classifications

| Type | Description | Key Requirements |
| --- | --- | --- |
| Type 1 | General fill | LL ≤ 80, PI ≤ 55, max particle 150mm |
| Type 2.1 | Structural fill (high quality) | LL ≤ 40, PI ≤ 20, CBR ≥ 8, max particle 75mm |
| Type 2.2 | Structural fill (moderate) | LL ≤ 50, PI ≤ 25, CBR ≥ 5, max particle 75mm |
| Type 2.3 | Bridge approach fill | LL ≤ 35, PI ≤ 12, CBR ≥ 15, max particle 37.5mm |
| Type 2.4 | Reinforced earth fill | LL ≤ 30, PI ≤ 6, specific grading |
| Type 3 | Rock fill | Max particle 2/3 layer thickness, no soil fines |
| Type 4 | Select fill (subgrade replacement) | CBR ≥ 15, specific grading envelope |

### 3.3 Platform Integration

Listing side:

Optional material specification fields: LL (Liquid Limit), PI (Plasticity Index), CBR, max particle size, grading

Upload field for test results (PDF) — Phase 2

Auto-classify material against MRTS05 types based on entered properties

Search/match side:

Filter by specification compliance: "Show me fill that meets MRTS05 Type 2.1"

Confidence badge: "Meets spec" (tested + compliant), "May meet spec" (partial data), "Unknown" (untested)

Example listing with spec data:

Clean Fill — Road Widening Project
├─ Material: Clean Fill / Earth
├─ Volume: 8,500 m³
├─ Test Results: LL=32, PI=14, CBR=12, Max particle=40mm
├─ MRTS05 Classification: ✓ Type 2.1 compliant
├─ Geotech Report: Available on request
└─ Confidence: ●●●●● VERIFIED

### 3.4 Other Specification Standards

| Standard | Application | Key Parameters |
| --- | --- | --- |
| MRTS05 | TMR road/bridge earthworks | LL, PI, CBR, grading, max particle |
| AS3798 | Residential/commercial fill | Compaction requirements, material classification |
| MRTS04 | General earthworks | Broader than MRTS05 |
| MRTS06 | Subgrade reinstatement | Specific to pavement works |
| DTMR RPEQ specifications | Project-specific | Vary by project |

## 4. Confidence & Risk Scoring

### 4.1 Confidence Levels

Each listing receives a confidence score based on available data:

| Level | Badge | Criteria |
| --- | --- | --- |
| ●●●●● VERIFIED | Green | Material tested by NATA lab + spec compliance confirmed + geotech report available |
| ●●●●○ HIGH | Green-amber | Material tested + basic spec data provided |
| ●●●○○ MEDIUM | Amber | Visual inspection only, or partial test data |
| ●●○○○ LOW | Amber-red | No testing, no spec data, but material type is low risk |
| ●○○○○ UNKNOWN | Red | No data, potentially contaminated, or from unknown source |

### 4.2 Risk Alerts

Automatic alerts triggered by listing characteristics:

| Trigger | Alert | Level |
| --- | --- | --- |
| Material from EMR/CLR listed area | "Source may be on contaminated land register — verify before accepting" | HIGH |
| Acid sulfate soil material | "ASSMP may be required — check AHD level at receiving site" | HIGH |
| Contaminated soil | "Soil Disposal Permit required — licensed facility only" | CRITICAL |
| Fire ant zone crossing | "BIP may be required for cross-zone movement" | HIGH |
| Wet material + structural fill spec | "Wet material may not meet moisture content requirements" | MEDIUM |
| Volume mismatch >50% | "Available volume significantly less than your requirement" | LOW |
| Expiring within 1 week | "This listing expires soon — act fast" | INFO |
| Untested material for spec-critical use | "Material is untested — request test results before committing" | MEDIUM |

### 4.3 Estimator Quick-Alert

For construction estimators, the platform provides a "Quick Cost Alert" summary:

┌─────────────────────────────────────────────────┐
│ 🔔 ESTIMATOR ALERT: Springfield Rise Estate     │
│                                                  │
│ 3 new listings match your 15,000m³ requirement  │
│                                                  │
│ Best option: Urban Dig Co, West End             │
│ Est. cost: $19.30/m³ delivered                  │
│ Risk: LOW (tested, same zone)                   │
│ Saving vs landfill: $3.4M                       │
│ Sustainability: 28t CO₂ avoided                 │
│                                                  │
│ ⚠ 1 option crosses fire ant zone               │
│ ⚠ 1 option expires in 5 days                   │
│                                                  │
│ [View Details] [Contact Supplier]               │
└─────────────────────────────────────────────────┘

## 5. Document Intelligence (Phase 3)

### 5.1 Geotech Report Upload & Parsing

Users can upload geotech or contamination reports (PDF). The platform uses AI to extract:

Material classification (LL, PI, CBR, grading curves)

Contamination results (heavy metals, PFAS, hydrocarbons)

NEPM compliance assessment

MRTS05 type classification

ASS indicators (pH, SCr results)

Output: Auto-populated listing fields + confidence score + risk alerts

### 5.2 Material Requirement Input

Users can describe their material requirement in plain language or by specification:

"I need MRTS05 Type 2.1 fill, 15,000m³"

"Clean fill for residential subdivision, minimum CBR 8"

"Rock fill for retaining wall, 150mm minus"

The platform matches these requirements against available listings with test data, showing compliance confidence.

## 6. Sustainability Analysis

### 6.1 Per-Transaction Sustainability Report

Every facilitated material exchange generates a sustainability impact report:

| Metric | Calculation | Unit |
| --- | --- | --- |
| Material diverted from landfill | Volume × bulk density | tonnes |
| Waste levy savings | Tonnes × levy rate | $ |
| Truck trips avoided | (Landfill distance - reuse distance) × loads | trips |
| Truck-km avoided | Trips avoided × avg distance | km |
| Diesel saved | Truck-km avoided × consumption rate | litres |
| CO₂ avoided | Diesel saved × 2.68 kg CO₂/L + embodied carbon of virgin material | tonnes CO₂ |
| Virgin material preserved | Volume of material that didn't need to be quarried | m³ |

### 6.2 Tender Sustainability Submission

For companies bidding on government contracts, Cut2Fill generates a downloadable sustainability statement:

─────────────────────────────────────────────────────
         CUT2FILL SUSTAINABILITY STATEMENT

Project: Springfield Rise Estate — Lot Pad Fill
Company: Ipswich Developments Pty Ltd
Date: 15 April 2026

MATERIAL REUSE SUMMARY
━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━
Material sourced via Cut2Fill:        15,000 m³
Material diverted from landfill:      25,500 tonnes
Waste levy savings:                   $2,932,500
Virgin quarry material avoided:       15,000 m³

TRANSPORT IMPACT
━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━
Truck trips (reuse vs landfill):      1,250 vs 2,500
Truck-km saved:                       62,500 km
Diesel saved:                         28,125 litres
CO₂ emissions avoided:               75.4 tonnes

COMPLIANCE
━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━
Material tested:                      ✓ NATA certified
MRTS05 compliance:                    ✓ Type 2.1
Fire ant zone crossing:               ✗ None
Contamination status:                 ✓ Clean (NEPM compliant)
Biosecurity compliance:               ✓ Same zone

This statement is generated by Cut2Fill — Australia's
construction material exchange platform.
cut2fill.com.au
─────────────────────────────────────────────────────

### 6.3 Why This Wins Tenders

QLD Government procurement increasingly requires sustainability criteria:

QLD Procurement Policy 2024 — requires consideration of environmental sustainability

TMR Sustainability Assessment — used in major road project tenders

Brisbane 2032 Sustainability Framework — circular economy is a specific target

IS Rating (Infrastructure Sustainability Council) — material sourcing and waste diversion are scored categories

A company that can demonstrate they sourced reused material instead of virgin quarry product, avoided X tonnes of CO₂, and saved Y truck-km scores higher on sustainability criteria even if their headline price is comparable or slightly higher. Cut2Fill makes this evidence automatic and verifiable.

## 7. Implementation Phases

| Phase | Features | Timeline |
| --- | --- | --- |
| MVP (current) | Listings, map, basic matching, fire ant zones, compliance tooltips, pricing badges | Delivered |
| Phase 2 | Cost calculator, routing distance, truck type selection, wet/dry flag, specification fields | Q3 2026 |
| Phase 3 | Document upload (geotech/contamination reports), AI parsing, MRTS05 auto-classification | Q4 2026 |
| Phase 4 | Sustainability reporting, tender submission generator, estimator alerts | Q1 2027 |
| Phase 5 | API for integration with estimating software (CostX, Candy, Buildsoft) | Q2 2027 |

## 8. CSIRO R&D Alignment

This feature set directly maps to CSIRO capabilities:

| Feature | CSIRO Research Area |
| --- | --- |
| Cost modelling (transport, fuel, time) | CSIRO Data61 — optimisation, logistics |
| Material specification matching | CSIRO Manufacturing — materials science |
| Contamination risk assessment | CSIRO Environment — contaminated land |
| Carbon accounting methodology | CSIRO Environment — emissions modelling |
| AI document parsing (geotech reports) | CSIRO Data61 — NLP, document intelligence |
| Circular economy metrics | CSIRO Manufacturing — industrial ecology |

This strengthens the Kick-Start application by demonstrating that the R&D challenge has genuine scientific depth beyond basic software development.

# Cut2Fill — Material Tracking, Compliance Automation & Long-Term Vision

Version: 1.0

Date: 23 March 2026

## 1. Material Tracking System

### 1.1 The Problem Today

Material movements on government projects currently require manual paper trails — docket books, Form B receipts, weighbridge tickets, and spreadsheets. This is:

Error-prone — handwritten dockets lost, illegible, or incomplete

Unverifiable — no independent record of where material actually went

Labour-intensive — admin staff manually reconciling dockets against claims

Non-compliant — auditors can't trace material chain of custody with confidence

Invisible — government has no real-time visibility on material flows across the region

### 1.2 Form B (TMR Lot Acceptance)

For Queensland Transport and Main Roads (TMR) projects, Form B — Lot Acceptance is the standard quality assurance document for earthworks. It records:

Material source and description

Test results (compaction, moisture, CBR, grading)

Lot identification (location, volume, layer)

Acceptance/rejection decision

Signed by RPEQ or authorised person

Form B is part of the TMR quality system under MRTS04/MRTS05 and is required for all government road and bridge earthworks. Similar documentation is required by councils and other government agencies under AS3798.

### 1.3 Cut2Fill Tracking Flow

PHASE 1 — Agreement & Registration
━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━
Supplier                              Receiver
   │                                     │
   ├─ Lists material on Cut2Fill         │
   │                                     ├─ Finds match, requests material
   │                                     │
   └────────── AGREEMENT ────────────────┘
               │
               ├─ Material type & spec confirmed
               ├─ Volume agreed
               ├─ Pricing agreed (free or quoted)
               ├─ Transport responsibility assigned
               ├─ Fire ant zone compliance checked ✓
               ├─ Permits identified (if required)
               └─ Digital agreement signed by both parties


PHASE 2 — Movement Tracking
━━━━━━━━━━━━━━━━━━━━━━━━━━━
                    ┌─────────────┐
                    │  Truck      │
                    │  Driver App │
                    │  (GPS)      │
                    └──────┬──────┘
                           │
        Load confirmed     │     Delivery confirmed
        at source ────────►├────► at destination
        (geofence)         │     (geofence)
                           │
                    Auto-logged:
                    ├─ Timestamp (load/unload)
                    ├─ GPS route taken
                    ├─ Source site (geofence match)
                    ├─ Destination site (geofence match)
                    ├─ Load count (running total)
                    ├─ Zone crossings detected
                    └─ Distance & duration


PHASE 3 — Compliance Certificate
━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━
When exchange is complete:
   │
   ├─ Total volume moved (load count × capacity)
   ├─ Material source verified (GPS)
   ├─ Material destination verified (GPS)
   ├─ Zone compliance confirmed (no unauthorised crossings)
   ├─ All loads timestamped and geolocated
   ├─ Test results attached (if uploaded)
   ├─ Agreement between parties on record
   │
   └─► CERTIFICATE OF COMPLIANCE generated
       ├─ Downloadable PDF for site files
       ├─ QR code for audit verification
       └─ Permanent record on platform

### 1.4 Truck Driver Experience

The driver doesn't need to do anything complex. They open the app, tap "Start Shift", and it runs in the background:

Start shift — driver selects the job (linked to an active agreement)

Auto-detect load — when GPS enters source site geofence + stationary for >5 min = loading

Auto-detect delivery — when GPS enters destination geofence + stationary for >5 min = unloading

Load counter — increments automatically, driver can correct if needed

End shift — summary of loads, distances, hours

No typing, no forms, no dockets — it just works

### 1.5 What This Replaces

| Current Process | Cut2Fill Replacement |
| --- | --- |
| Paper docket books | Automatic GPS load/unload logging |
| Manual load counting | Auto-increment with geofence detection |
| Weighbridge tickets (some sites) | Integrates with weighbridge data (Phase 4) |
| Form B manual preparation | Auto-populated from tracked data + test results |
| Spreadsheet reconciliation | Real-time dashboard showing loads, volume, progress |
| End-of-project audit folder | Digital compliance certificate with full trail |

## 2. Agreement System

### 2.1 Digital Agreement Between Parties

When a match is confirmed, both parties digitally agree to terms:

| Field | Detail |
| --- | --- |
| Supplier | Company, contact, site address |
| Receiver | Company, contact, site address |
| Material | Type, specification (if applicable), test status |
| Volume | Agreed quantity (m³) |
| Pricing | Free or agreed rate |
| Transport | Who arranges — supplier, receiver, or third party |
| Timing | Start date, end date, permitted hours |
| Access | Site access conditions, safety requirements |
| Compliance | Fire ant zone status, permits identified, testing requirements |
| Acceptance criteria | What constitutes acceptable material at receiving end |

This isn't a legal contract — it's a mutual acknowledgement that both parties understand the terms and compliance requirements. It creates a record that can be referenced if issues arise.

### 2.2 Dispute Prevention

The agreement + tracking combination prevents the most common disputes:

"They sent less than agreed" → GPS-verified load count

"The material wasn't what was described" → Test results on record, acceptance criteria documented

"They dumped outside hours" → Timestamped delivery records

"We never agreed to that price" → Digital agreement with both parties' confirmation

## 3. Compliance Certificates

### 3.1 Certificate Contents

═══════════════════════════════════════════════════════════
              CUT2FILL MATERIAL MOVEMENT CERTIFICATE
═══════════════════════════════════════════════════════════

Certificate ID:     C2F-2026-00142
Date Issued:        30 June 2026
Agreement Ref:      AGR-2026-00089

SOURCE
━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━
Company:            Urban Dig Co Pty Ltd
Project:            Montague Markets Tower, West End
Contact:            [Name] — 0400 123 456
Material:           Clean Fill / Earth
Test Results:       LL=32, PI=14, CBR=12 (NATA Cert #12345)

DESTINATION
━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━
Company:            Ipswich Developments Pty Ltd
Project:            Springfield Rise Estate, Springfield
Contact:            [Name] — 0422 888 999
Specification:      MRTS05 Type 2.1 (confirmed compliant)

MOVEMENT SUMMARY
━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━
Period:             1 April 2026 — 28 June 2026
Total Loads:        500
Total Volume:       6,000 m³ (estimated from load count)
Transport:          Truck & dog (12m³ capacity)
Average Distance:   28.4 km per trip

COMPLIANCE
━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━
Fire Ant Zone:      Source Zone 1, Destination Zone 2
Zone Crossing:      ✓ Compliant — movement Zone 1 → Zone 2 permitted
BIP Required:       No (inward movement to lower restriction zone)
EMR/CLR Check:      ✓ Source site not listed
Contamination:      ✓ Clean — NEPM compliant (test certificate attached)
Acid Sulfate:       N/A — source above 5m AHD

SUSTAINABILITY IMPACT
━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━
Material diverted from landfill:    10,200 tonnes
Waste levy saving:                  $1,173,000
Truck-km avoided (vs nearest tip): 31,250 km
CO₂ emissions avoided:             37.5 tonnes
Virgin quarry material preserved:   6,000 m³

VERIFICATION
━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━
This certificate was generated from GPS-verified
movement data recorded through the Cut2Fill platform.

QR Code: [████████] — Scan to verify online
URL: cut2fill.com.au/verify/C2F-2026-00142

═══════════════════════════════════════════════════════════

### 3.2 Certificate Use Cases

| Audience | How They Use It |
| --- | --- |
| Site engineer | Attach to Form B lot acceptance documentation |
| Project manager | Include in monthly progress reports |
| Estimator | Reconcile actual vs estimated cartage costs |
| Council compliance officer | Verify fill placement approvals were met |
| TMR quality auditor | Trace material source and chain of custody |
| Tender submission | Demonstrate sustainability credentials |
| BQCC inspector | Verify fire ant zone compliance for material movements |
| Environmental auditor | Confirm no contaminated material was placed inappropriately |

## 4. Free Tier Strategy — Useable From Day One

### 4.1 Core Principle

The platform must deliver value the moment someone lands on it, without signing up, without paying, without a sales call.

### 4.2 What's Free Forever

| Feature | Detail |
| --- | --- |
| Browse all listings | See every active listing on the map — no login wall |
| View registered facilities | Every quarry, tip, transfer station, recycler mapped |
| Fire ant zone overlay | Toggle zones on/off, see which zone a listing is in |
| Material compliance tooltips | QLD regulatory guidance for all 8 material types |
| Basic matching | Click "Find Matches" on any listing |
| Cost comparison (basic) | Straight-line distance estimate, levy comparison |
| Post listings | Register (free) and post unlimited listings |
| Contact suppliers/receivers | Direct contact details visible |

### 4.3 What's Premium (Phase 2+)

| Feature | Value Proposition | Price Point |
| --- | --- | --- |
| True cost calculator | Routing distance, truck selection, fuel, time — real numbers | $50/month |
| Specification matching | Match against MRTS/AS specs, filter by material properties | $50/month |
| Material tracking (GPS) | Automated load counting, route logging | $200/month per active agreement |
| Compliance certificates | Downloadable PDF with full chain of custody | $50 per certificate |
| Sustainability reports | Tender-ready impact statements | Included with tracking |
| Estimator alerts | Push notifications when material matches your requirements | $50/month |
| API access | Integration with estimating software | $500/month |
| Verified supplier badge | Platform-verified, builds trust | $200/year |
| Priority listing | Appear first in search results | $100/month |

### 4.4 The Conversion Logic

Free users see enough to understand the value:

"This exchange would save you approximately $X vs landfill" (basic estimate)

"Upgrade for exact routing cost, specification matching, and compliance certificates"

The gap between the basic estimate and the precise calculation is where the subscription earns its keep

A $50/month subscription that saves a single project $50,000 in one transaction is an absurd ROI. The product sells itself once people see real numbers.

## 5. Material Treatment at Scale

### 5.1 The Insight

Not all material needs to go to landfill, and not all material is ready for reuse in its current state. There's a middle ground: treatment.

Currently, low-quality materials are sent to landfill because:

Nobody knows where treatment facilities exist

Treatment costs are unclear

The logistics of getting material to treatment and then to its final destination are complex

There's no way to compare treatment + reuse vs straight-to-landfill costs

### 5.2 Treatment Options

| Material Issue | Treatment | Outcome | Facility Type |
| --- | --- | --- | --- |
| Oversized particles | Screening, crushing | Meets grading spec | Mobile or fixed crusher |
| High plasticity (PI too high) | Lime stabilisation | Reduced PI, improved CBR | On-site or mobile plant |
| Contaminated (hydrocarbons) | Bioremediation, thermal desorption | Below NEPM limits | Licensed treatment facility |
| Contaminated (PFAS) | Immobilisation, thermal treatment | Below guideline values | HiQ Yatala (QLD's only PFAS facility) |
| Acid sulfate potential | Lime treatment, neutralisation | pH stabilised | On-site with ASSMP |
| Wet material | Drying (stockpile, lime addition) | Meets moisture spec | On-site or dedicated pad |
| Mixed C&D | Sorting, screening | Separated recyclable streams | Licensed C&D recycler |

### 5.3 Cut2Fill's Role in Treatment

The platform enables a three-way exchange:

SOURCE ──────► TREATMENT FACILITY ──────► DESTINATION
(low quality    (upgrade material        (receives material
 material)       to specification)        meeting spec)

Example:

Contractor A has 5,000m³ of clay fill (PI=35, too plastic for MRTS05 Type 2.1)

Lime stabilisation plant can treat it to PI<20 for $25/m³

Contractor B needs 5,000m³ of Type 2.1 fill, willing to pay $15/m³

Cost comparison:

Without treatment: Contractor A pays $115/t levy to landfill = ~$977,500. Contractor B buys virgin quarry material at $30/m³ = $150,000. Total: $1,127,500

With Cut2Fill treatment pathway: Treatment $125,000 + cartage. Contractor B gets fill at $15/m³ = $75,000. Total: ~$300,000. Saving: $827,500

### 5.4 Whole-of-Life Material Decisions

This is where Cut2Fill becomes genuinely valuable to engineers:

"Should I use expensive, perfectly-graded imported fill? Or cheaper, treated local fill that achieves the same performance?"

The platform helps engineers make whole-of-life decisions by comparing:

| Factor | Virgin Quarry Material | Treated Reused Material |
| --- | --- | --- |
| Material cost | $25-40/m³ | $0-25/m³ (treatment cost) |
| Transport | Long-haul from quarry | Potentially shorter from nearby site |
| Embodied carbon | High (quarrying, crushing, transport) | Lower (no extraction) |
| Landfill diversion | None | Full volume diverted |
| Waste levy avoided | None | $115/t saved by source |
| Specification compliance | Guaranteed by quarry | Verified by testing post-treatment |
| Sustainability score | Low | High |

Sometimes the treated reused material is genuinely cheaper. Sometimes it's slightly more expensive but wins on sustainability. Sometimes the virgin material is the right call. Cut2Fill gives the data to make that decision with confidence rather than defaulting to "just send it to the tip."

## 6. Platform Evolution Roadmap

NOW ──────────────────────────────────────────────────► FUTURE

PHASE 1: MAP & MATCH (MVP — delivered)
├─ Listings, map, matching, fire ant zones
├─ Compliance tooltips, pricing badges
├─ Registered facilities layer
└─ FREE — useable immediately

PHASE 2: COST & SPECIFICATION (Q3-Q4 2026)
├─ True cost calculator with routing
├─ Material specification fields
├─ Confidence/risk scoring
├─ Premium subscriptions launch
└─ CSIRO Kick-Start collaboration begins

PHASE 3: INTELLIGENCE (Q1-Q2 2027)
├─ Geotech/contamination report upload
├─ AI document parsing
├─ Treatment pathway recommendations
├─ Sustainability reporting
└─ Government pilot (2-3 councils)

PHASE 4: TRACKING (Q3-Q4 2027)
├─ Digital agreements between parties
├─ Truck driver GPS tracking app
├─ Automated load counting
├─ Compliance certificate generation
├─ Form B auto-population
└─ Government data feed

PHASE 5: ECOSYSTEM (2028+)
├─ API for estimating software integration
├─ Weighbridge data integration
├─ Treatment facility booking
├─ National expansion (NSW, VIC regulations)
├─ EOW code advocacy (data-driven submission)
├─ Real-time regional material balance dashboard (government)
└─ Automated material passport (blockchain optional)

## 7. Why Landfills Aren't Always the Answer

### 7.1 The Default Behaviour Problem

The construction industry defaults to landfill because:

It's simple — call a tip truck, send it to the nearest licensed site

It's habitual — "that's how we've always done it"

The alternatives are unknown — "I didn't know there was a treatment option"

Time pressure — finding a reuse option takes effort; landfill takes a phone call

### 7.2 When Reuse/Treatment Is Better Than Landfill

| Scenario | Landfill | Reuse/Treatment | Winner |
| --- | --- | --- | --- |
| 10,000m³ clean fill, receiver 15km away | $1.15M levy + $80K cartage | $0 + $60K cartage | Reuse saves $1.17M |
| 3,000m³ clay (PI too high), lime plant 20km away | $345K levy + $50K cartage | $75K treatment + $40K cartage | Treatment saves $280K |
| 500m³ contaminated soil, PFAS detected | $120K levy + gate fee + $30K cartage | $90K treatment (HiQ Yatala) + $20K cartage | Treatment saves $40K + material recovered |
| 5,000m³ mixed C&D | $575K levy + $60K cartage | $125K sorting/recycling + $40K cartage | Recycling saves $470K |

### 7.3 Cut2Fill Makes the Alternative Visible

The platform doesn't tell people they can't use a landfill. It shows them what the alternatives are, what they cost, and what the compliance requirements are — then lets them make an informed decision. Most of the time, the numbers speak for themselves.

## 8. Data Value to Government

### 8.1 What Government Gets From Cut2Fill Data

| Data | Government Use |
| --- | --- |
| Real-time material flows across SEQ | Infrastructure planning, waste strategy monitoring |
| Volume of material diverted from landfill | Waste reduction target tracking |
| Fire ant zone compliance rates | Biosecurity enforcement intelligence |
| Material pricing trends | Market analysis, policy impact assessment |
| Regional supply/demand imbalance | Identify where infrastructure investment is needed |
| Treatment facility utilisation | Capacity planning |
| Transport patterns | Road planning, truck route management |
| Carbon emissions avoided | Climate action reporting |

### 8.2 The Government Partnership Pitch

"We're building the data infrastructure that your waste strategy needs but doesn't currently have. Every material exchange on Cut2Fill generates data that helps you track progress against circular economy targets, enforce biosecurity compliance, and plan infrastructure investment. We're not asking for a handout — we're offering a data partnership where the platform serves industry and government simultaneously."

This is not a startup asking government for money. This is a platform offering government visibility they can't currently get anywhere else, in exchange for endorsement, data access, and eventually licensing fees.